Chapter 15 Roof Assemblies and Rooftop Structures

CALIFORNIA BUILDING CODE — MATRIX ADOPTION TABLE
CHAPTER 15 — ROOF ASSEMBLIES AND ROOFTOP STRUCTURES

(Matrix Adoption Tables are nonregulatory, intended only as an aid to the code user.
See Chapter 1 for state agency authority and building applications.)

| Adopting agency | BSC | BSC- CG | SFM | HCD |  |  | DSA |  |  | OSHPD |  |  |  |  |  |  | BSCC | DPH | AGR | DWR | CEC | CA | SL | SLC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1 | 2 | 1/AC | AC | SS | SS/CC | 1 | 1R | 2 | 3 | 4 | 5 | 6 |  |  |  |  |  |  |  |  |
| Adopt entire chapter |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  | X |  |  |  |  |  |  |  |  |
| Adopt entire chapter as amended (amended sections listed below) | X |  |  | X | X |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  |  |  |  |
| Adopt only those sections that are listed below |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Chapter / Section |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1501 |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1502.1 |  |  |  | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1502.2 |  |  |  | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1503.3 — 1503.4 |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1505 — 1507 |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1507.3.10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1507.7.8 |  |  |  |  |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  |  |  |  |
| 1509 — 1511 |  |  | X |  |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  |  |  |  |
| 1511.9.6 |  |  |  |  |  | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1511.10 — 1511.10.1 | X |  |  | X | X |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  |  |  |  |
| 1512 |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1513 |  |  |  |  |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  |  |  |  |

The state agency does not adopt sections identified by the following symbol: †

The Office of the State Fire Marshal's adoption of this chapter or individual sections is applicable to structures regulated by other state agencies pursuant to Section 1.11.

User notes:

About this chapter: Chapter 15 provides minimum requirements for the design and construction of roof assemblies and rooftop structures. The criteria address the weather-protective barrier at the roof and, in most circumstances, a fire-resistant barrier. The chapter is largely prescriptive in nature and is based on decades of experience with various traditional materials, but it also recognizes newer products. Section 1511 addresses rooftop structures, which include penthouses, tanks, towers and spires. Rooftop penthouses larger than prescribed in this chapter must be treated as a story under Chapter 5.

ICC code development note: Code change proposals to sections preceded by the designation [BF] or [P] will be considered by one of the code development committees meeting during the 2024 (Group A) Code Development Cycle. All other code change proposals will be considered by a code development committee meeting during the 2025 (Group B) Code Development Cycle.

Section 1501 General

Diagram

UpCodes Diagrams (2)

Overhead Service Conductors: Above Roofs (< 1000V)

Overhead Service Conductors: Vertical Clearance (< 1000V)

1501.1 Scope

The provisions of this chapter shall govern the design, materials, construction and quality of roof assemblies, and rooftop structures.

Section 1502 Roof Drainage

Building products (2)

FOAMULAR® NGX® 400/600/1000

Owens Corning

FOAMULAR® NGX® 404RB/604RB

Owens Corning

[P] 1502.1 General

Design and installation of roof drainage systems shall comply with this section, Section 1611 of this code and Chapter 11 of the California Plumbing Code .

Building products (2)

FLOFORCE® High Performance Roof Drain - Z100F

Zurn

High Performance Promenade Deck Drain with Rotatable Frame and Heel-Proof Grate - Z150F

Zurn

[P] 1502.2 Secondary (Emergency Overflow) Drains or Scuppers

Where roof drains are required, secondary (emergency overflow) roof drains or scuppers shall be provided where the roof perimeter construction extends above the roof in such a manner that water will be entrapped if the primary drains allow buildup for any reason. The installation and sizing of secondary emergency overflow drains, leaders and conductors shall comply with Section 1611 of this code and Chapter 11 of the California Plumbing Code.

1502.3 Gutters

Gutters and leaders placed on the outside of buildings, other than Group R-3, private garages and buildings of Type V construction, shall be of noncombustible material or not less than Schedule 40 plastic pipe.

Section 1503 Weather Protection

Diagram

UpCodes Diagrams (5)

Exterior Wall Flashing (2021)

Roof Flashing Locations

Ice Barriers: Roofing

Roof Tile Valley Flashing

Crickets and Saddles

1503.1 General

Roof decks shall be covered with approved roof coverings secured to the building or structure in accordance with the provisions of this chapter. Roof coverings shall be designed in accordance with this code, and installed in accordance with this code and the manufacturer's approved instructions.

1503.2 Flashing

Diagram

Flashing shall be installed in such a manner so as to prevent water from entering the wall and roof through joints in copings, through moisture-permeable materials and at intersections with parapet walls and other penetrations through the roof plane.

UpCodes Diagrams (6)

Primary

Kick-Out Flashing

Weatherproofing Fire Rated vs. Other Parapet Walls

Roof Curb

Exterior Wall Flashing (2021)

Parapet Const.

Vent Pipe: Roof Termination Sizing

1503.2.1 Locations

Diagram

Flashing shall be installed at wall and roof intersections, at gutters, wherever there is a change in roof slope or direction and around roof openings. Where flashing is of metal, the metal shall be corrosion resistant with a thickness of not less than 0.019 inch (0.483 mm) (No. 26 galvanized sheet).

UpCodes Diagrams (2)

Primary

Roof Flashing Locations

Exterior Wall Flashing (2021)

1503.3 Parapet Walls

Diagram

Parapet walls shall be coped or covered in accordance with Sections 1503.3.1 and 1503.3.2. The top surface of the parapet wall shall provide positive drainage.

UpCodes Diagram (1)

Parapet Const.

1503.3.1 Fire-Resistance-Rated Parapet Walls

Diagram

Parapet walls required by Section 705.12 shall be coped or covered with weather-proof materials of a width not less than the thickness of the parapet wall such that the fire-resistance rating of the wall is not decreased.

UpCodes Diagram (1)

Primary

Weatherproofing Fire Rated vs. Other Parapet Walls

1503.3.2 Other Parapet Walls

Diagram

Parapet walls meeting one of the exceptions in Section 705.12 shall be coped or covered with weatherproof materials of a width not less than the thickness of the parapet wall.

UpCodes Diagram (1)

Primary

Weatherproofing Fire Rated vs. Other Parapet Walls

1503.4 Attic and Rafter Ventilation

Diagram

Intake and exhaust vents for ventilation of attic and enclosed rafter assemblies shall be provided in accordance with Section 1202.2 and the vent product manufacturer's installation instructions.

Exception: Unvented attic and unvented enclosed rafter assemblies in accordance with Section 1202.3.

UpCodes Diagrams (3)

Attic Ventilation

Example: Ventilation of Attics - Conventional Attics

Example: Ventilation of Attics - Clg. Applied Directly to Roof Rafters

1503.5 Crickets and Saddles

Diagram

A cricket or saddle shall be installed on the ridge side of any chimney or penetration greater than 30 inches (762 mm) wide as measured perpendicular to the slope. Cricket or saddle coverings shall be sheet metal or of the same material as the roof covering.

Exception: Unit skylights installed in accordance with Section 2405.5 and flashed in accordance with the manufacturer's instructions shall be permitted to be installed without a cricket or saddle.

UpCodes Diagram (1)

Primary

Crickets and Saddles

Section 1504 Performance Requirements

1504.1 Wind Resistance of Roofs

Roof decks and roof coverings shall be designed in accordance with Section 1504.

1504.2 Wind Resistance of Asphalt Shingles

Diagram

Asphalt shingles shall be tested in accordance with ASTM D7158. Asphalt shingles shall meet the classification requirements of Table 1504.2 for the appropriate maximum basic wind speed. Asphalt shingle packaging shall bear a label to indicate compliance with ASTM D7158 and the required classification in Table 1504.2.

Exception: Asphalt shingles not included in the scope of ASTM D7158 shall be tested and labeled in accordance with ASTM D3161. Asphalt shingle packaging shall bear a label to indicate compliance with ASTM D3161 and the required classification in Table 1504.2.

TABLE 1504.2—CLASSIFICATION OF STEEP SLOPE ROOF SHINGLES TESTED IN ACCORDANCE WITH ASTM D3161 OR D7158

| MAXIMUM BASIC WIND SPEED, V, FROM FIGURES 1609.3(1)—(4) OR ASCE 7 (mph) | MAXIMUM ALLOWABLE STRESS DESIGN WIND SPEED, Vasd, FROM Table 1609.3.1 (mph) | ASTM D7158a CLASSIFICATION | ASTM D3161 or UL 7103 CLASSIFICATION |
| --- | --- | --- | --- |
| 110 | 85 | D, G or H | A, D or F |
| 116 | 90 | D, G or H | A, D or F |
| 129 | 100 | G or H | A, D or F |
| 142 | 110 | G or H | F |
| 155 | 120 | G or H | F |
| 168 | 130 | H | F |
| 181 | 140 | H | F |
| 194 | 150 | H | F |

For SI: 1 foot = 304.8 mm, 1 mph = 0.447 m/s.

- The standard calculations contained in ASTM D7158 assume Exposure Category B or C and building height of 60 feet or less. Additional calculations are required for conditions outside of these assumptions.

UpCodes Diagrams (2)

Proper Location for Asphalt Shingle Fasteners

Asphalt Shingles and Fasteners

1504.3 Wind Resistance of Clay and Concrete Tile

Wind loads on clay and concrete tile roof coverings shall be in accordance with Section 1609.6.

1504.3.1 Testing

Testing of concrete and clay roof tiles shall be in accordance with Sections 1504.3.1.1, 1504.3.1.2 and 1504.3.1.3.

1504.3.1.1 Overturning Resistance

Concrete and clay roof tiles shall be tested to determine their resistance to overturning due to wind in accordance with Chapter 15 and either SBCCI SSTD 11 or ASTM C1568.

1504.3.1.2 Wind Tunnel Testing

Where concrete and clay roof tiles do not satisfy the limitations in Chapter 16 for rigid tile, a wind tunnel test shall be used to determine the wind characteristics of the concrete or clay tile roof covering in accordance with Chapter 15 and either SBCCI SSTD 11 or ASTM C1569.

1504.3.1.3 Air Permeability Testing

The lift coefficient for concrete and clay tile shall be 0.2 or shall be determined in accordance with SBCCI SSTD 11 or ASTM C1570.

1504.4 Wind Resistance of Nonballasted Roofs

Roof coverings installed on roofs in accordance with Section 1507 that are mechanically attached or adhered to the roof deck shall be designed to resist the design wind load pressures for components and cladding in accordance with Section 1609.6.2. The wind load on the roof covering shall be permitted to be determined using allowable stress design.

Building products (4)

FOAMULAR® & FOAMULAR® NGX® Thermapink® 25 Tapered

Owens Corning

FOAMULAR® NGX® 400/600/1000

Owens Corning

FOAMULAR® NGX® 404RB/604RB

Owens Corning

FOAMULAR® NGX® THERMAPINK® 25 XPS

Owens Corning

1504.4.1 Other Roof Systems

Built-up, modified bitumen, fully adhered or mechanically attached single-ply roof systems, metal panel roof systems applied to a solid or closely fitted deck and other types of membrane roof coverings shall be tested in accordance with FM 4474, UL 580 or UL 1897.

1504.4.2 Structural Metal Panel Roof Systems

Where the metal roof panel functions as the roof deck and roof covering and it provides both weather protection and support for loads, the structural metal panel roof system shall comply with this section. Structural standing-seam metal panel roof systems shall be tested in accordance with ASTM E1592 or FM 4474. Structural through-fastened metal panel roof systems shall be tested in accordance with ASTM E1592, FM 4474 or UL 580.

Exceptions:

- Metal roofs constructed of cold-formed steel shall be permitted to be designed and tested in accordance with the applicable referenced structural design standard in Section 2204.1.

- Metal roofs constructed of aluminum shall be permitted to be designed and tested in accordance with the applicable referenced structural design standard in Section 2002.1.

1504.4.3 Metal Roof Shingles

Metal roof shingles applied to a solid or closely fitted deck shall be tested in accordance with ASTM D3161, FM 4474, UL 580 or UL 1897. Metal roof shingles tested in accordance with ASTM D3161 shall meet the classification requirements of Table 1504.2 for the appropriate maximum basic wind speed and the metal shingle packaging shall bear a label to indicate compliance with ASTM D3161 and the required classification in Table 1504.2.

1504.4.4 Slate Shingles

Slate shingles shall be tested in accordance with ASTM D3161. Slate packaging shall bear a label indicating compliance with ASTM D3161 and the required classification in Table 1504.2.

1504.5 Ballasted Low-Slope Single-Ply Roof Systems

Ballasted low-slope single-ply roof system coverings installed in accordance with Section 1507.12 shall be designed in accordance with ANSI/SPRI RP-4.

1504.6 Edge Systems for Low-Slope Roofs

Metal edge systems, except gutters and counterflashing, installed on built-up, modified bitumen and single-ply roof systems on a low-slope roof shall be designed and installed for wind loads in accordance with Chapter 16 and tested for resistance in accordance with Test Methods RE-1, RE-2 and RE-3 of ANSI/SPRI ES-1, except basic wind speed, V, shall be determined from Figures 1609.3(1)through 1609.3(4), as applicable.

1504.6.1 Gutter Securement for Low-Slope Roofs

Gutters that are used to secure the perimeter edge of the roof membrane on low-slope built-up, modified bitumen, and single-ply roofs, shall be designed, constructed and installed to resist wind loads in accordance with Section 1609 and shall be tested in accordance with Test Methods G-1 and G-2 of SPRI GT-1.

1504.7 Impact Resistance

Roof coverings installed on low-slope roofs in accordance with Section 1507 shall resist impact damage based on the results of tests conducted in accordance with ASTM D3746, ASTM D4272 or the "Resistance to Foot Traffic Test" in FM 4470.

Building products (4)

FOAMULAR® & FOAMULAR® NGX® Thermapink® 25 Tapered

Owens Corning

FOAMULAR® NGX® 400/600/1000

Owens Corning

FOAMULAR® NGX® 404RB/604RB

Owens Corning

FOAMULAR® NGX® THERMAPINK® 25 XPS

Owens Corning

1504.8 Wind Resistance of Aggregate-Surfaced Roofs

Parapets shall be provided for aggregate surfaced roofs and shall comply with Table 1504.8. Such parapets shall be provided on the perimeter of the roof at all exterior sides except where an adjacent wall extends above the roof to a height at least equivalent to that required for the parapet. For roofs with differing surface elevations due to slope or sections at different elevations, the minimum parapet height shall be determined based on each roof surface elevation, and at no point shall the parapet height be less than that required by Table 1504.8.

Exception: Ballasted single-ply roof coverings shall be designed and installed in accordance with Section 1504.5.

TABLE 1504.8—MINIMUM REQUIRED PARAPET HEIGHT (INCHES) FOR AGGREGATE SURFACED ROOFSa, b, c, d, e

| AGGREGATE SIZE | MEAN ROOF HEIGHT (feet) | WIND EXPOSURE AND BASIC WIND SPEED, V (MPH) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Exposure B |  |  |  |  |  |  |  |  | Exposure Cf |  |  |  |  |  |  |  |  |
|  |  | ≤ 95 | 100 | 105 | 110 | 115 | 120 | 130 | 140 | 150 | ≤ 95 | 100 | 105 | 110 | 115 | 120 | 130 | 140 | 150 |
| ASTM D1863 (No. 7 or No. 67) | 15 | 2 | 2 | 2 | 2 | 12 | 12 | 16 | 20 | 24 | 2 | 13 | 15 | 18 | 20 | 23 | 27 | 32 | 37 |
|  | 20 | 2 | 2 | 2 | 2 | 12 | 14 | 18 | 22 | 26 | 12 | 15 | 17 | 19 | 22 | 24 | 29 | 34 | 39 |
|  | 30 | 2 | 2 | 2 | 13 | 15 | 17 | 21 | 25 | 30 | 14 | 17 | 19 | 22 | 24 | 27 | 32 | 37 | 42 |
|  | 50 | 12 | 12 | 14 | 16 | 18 | 21 | 25 | 30 | 35 | 17 | 19 | 22 | 25 | 28 | 30 | 36 | 41 | 47 |
|  | 100 | 14 | 16 | 19 | 21 | 24 | 27 | 32 | 37 | 42 | 21 | 24 | 26 | 29 | 32 | 35 | 41 | 47 | 53 |
|  | 150 | 17 | 19 | 22 | 25 | 27 | 30 | 36 | 41 | 46 | 23 | 26 | 29 | 32 | 35 | 38 | 44 | 50 | 56 |
| ASTM D1863 (No. 6) | 15 | 2 | 2 | 2 | 2 | 12 | 12 | 12 | 15 | 18 | 2 | 2 | 2 | 13 | 15 | 17 | 22 | 26 | 30 |
|  | 20 | 2 | 2 | 2 | 2 | 12 | 12 | 13 | 17 | 21 | 2 | 2 | 12 | 15 | 17 | 19 | 23 | 28 | 32 |
|  | 30 | 2 | 2 | 2 | 2 | 12 | 12 | 16 | 20 | 24 | 2 | 12 | 14 | 17 | 19 | 21 | 26 | 31 | 35 |
|  | 50 | 12 | 12 | 12 | 12 | 14 | 16 | 20 | 24 | 28 | 12 | 15 | 17 | 19 | 22 | 24 | 29 | 34 | 39 |
|  | 100 | 12 | 12 | 14 | 16 | 19 | 21 | 26 | 30 | 35 | 16 | 18 | 21 | 24 | 26 | 29 | 34 | 39 | 45 |
|  | 150 | 12 | 14 | 17 | 19 | 22 | 24 | 29 | 34 | 39 | 18 | 21 | 23 | 26 | 29 | 32 | 37 | 43 | 48 |

For SI: 1 inch = 25.4 mm, 1 foot = 304.8 mm, 1 mile per hour = 0.447 m/s.

- Parapet height is measured vertically from the top surface of the coping down to the surface of the roof covering in the field of the roof adjacent to the parapet and outbound of any cant strip.

- Interpolation shall be permitted for wind speed, mean roof height and parapet height. Extrapolation is not permitted.

- Basic wind speed, V, and wind exposure shall be determined in accordance with Section 1609.

- Where the minimum required parapet height is indicated to be 2 inches (51 mm), a gravel stop shall be permitted and shall extend not less than 2 inches (51 mm) from the roof surface and not less than the height of the aggregate.

- The tabulated values apply only to conditions where the topographic factor (Kzt) determined in accordance with Chapter 26 of ASCE 7 is 1.0 or where Kzt is incorporated in the basic wind speed in Section 1609.

- For Exposure D, add 8 inches (203 mm) to the parapet height required for Exposure C and the parapet height shall not be less than 12 inches (305 mm).

Section 1505 Fire Classification

[BF] 1505.1 General

Diagram

Fire classification of roof assemblies shall be in accordance with Section 1505. The minimum fire classification of roof assemblies installed on buildings shall comply with Table 1505.1 based on type of construction of the building. Class A, B and C roof assemblies and roof coverings required to be listed by this section shall be tested in accordance with ASTM E108 or UL 790. In addition, fire-retardant-treated woodroof coverings shall be tested in accordance with ASTM D2898; fire-retardant-treated shingles and shakes shall comply with Section 1505.6.

Exception: Skylights and sloped glazing that comply with Chapter 24 or Section 2610.

[BF] TABLE 1505.1—MINIMUM ROOF ASSEMBLY CLASSIFICATION FOR TYPES OF CONSTRUCTIONa, b

| IA | IB | IIA | IIB | IIIA | IIIB | IV | VA | VB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B | B | B | Cc | B | Cc | B | B | Cc |

For SI: 1 foot = 304.8 mm, 1 square foot = 0.0929 m2.

- Unless otherwise required in accordance with the California Wildland-Urban Interface Code or due to the location of the building within a fire district in accordance with Appendix D.

- Nonclassified roof coverings shall be permitted on buildings of Group U occupancies, where there is a minimum fire-separation distance of 6 feet measured from the leading edge of the roof.

- Buildings that are not more than two stories above grade plane and having not more than 6,000 square feet of projected roof area and where there is a minimum 10-foot fire-separation distance from the leading edge of the roof to a lot line on all sides of the building, except for street fronts or public ways, shall be permitted to have roofs of No. 1 cedar or redwood shakes and No. 1 shingles constructed in accordance with Section 1505.7.

UpCodes Diagrams (3)

Primary

Roof Assembly Fire Classification

Class A Roof Assemblies

Class A Roof Assemblies

1505.1.1 Roofing Requirements Within Fire Hazard Severity Zones or in the Wildland-Urban Interface (WUI)

Diagram

Roofing requirements for structures located within Fire Hazard Severity Zones or in the Wildland-Urban Interface (WUI) shall also comply with Section 705A.

UpCodes Diagram (1)

Roof Coverings: Wildland Urban Interface

1505.1.2 Roof Coverings Within All Other Areas Other Than Fire Hazard Severity Zones or a Wildland-Urban Interface (WUI)

The entire roof covering of every existing structure where more than 50 percent of the total roof area is replaced within any one-year period, the entire roof covering of every new structure, and any roof covering applied in the alteration, repair or replacement of the roof of every existing structure, shall be a fire-retardant roof covering that is at least Class C fire classification.

[BF] 1505.2 Class A Roof Assemblies

Assembly

Diagram

Class A roof assemblies are those that are effective against severe fire test exposure. Class A roof assemblies and roof coverings shall be listed and identified as Class A by an approved testing agency. Class A roof assemblies shall be permitted for use in buildings or structures of all types of construction.

Exceptions:

- Class A roof assemblies include those with coverings of brick, masonry or an exposed concrete roof deck.

- Class A roof assemblies also include ferrous or copper shingles or sheets, metal sheets and shingles, clay or concrete roof tile or slate installed on noncombustible decks or ferrous, copper or metal sheets installed without a roof deck on noncombustible framing.

- Class A roof assemblies include minimum 16 ounce per square foot (0.0416 kg/m2) copper sheets installed over combustible decks.

- Class A roof assemblies include slate installed over ASTM D226, Type II or ASTM D4869, Type IV underlayment over combustible decks.

UpCodes Diagrams (4)

Primary

Class A Roof Assemblies

Primary

Class A Roof Assemblies

Slate Roofing

Slate Roof: Shingle Layout Plan

Assemblies (10)

17-1.1: 2hr, Steel/Conc.

17-1.1: 3hr, Steel/Conc.

18-1.1: 2hr & 3hr, Steel/Conc.

20-1.1: 2hr, Steel/Conc.

10-1.1: 2hr, Steel

10-1.1: 1hr, Steel

11-1.1: 1hr, Steel

11-1.1: 2hr, Steel

11-1.1: 2hr, Steel - Options 2 & 3

12-1.1: 2hr, Steel

[BF] 1505.3 Class B Roof Assemblies

Assembly

Diagram

Class B roof assemblies are those that are effective against moderate fire-test exposure. Class B roof assemblies and roof coverings shall be listed and identified as Class B by an approved testing agency.

UpCodes Diagrams (4)

Fire Walls: Vertical Continuity - Exception 4

Fire Walls: Vertical Continuity - Exception 3

Fire Walls: Vertical Continuity - Exception 2

Fire Walls: Vertical Continuity - Base Condition

Assemblies (10)

17-1.1: 2hr, Steel/Conc.

17-1.1: 3hr, Steel/Conc.

18-1.1: 2hr & 3hr, Steel/Conc.

20-1.1: 2hr, Steel/Conc.

10-1.1: 2hr, Steel

10-1.1: 1hr, Steel

11-1.1: 1hr, Steel

11-1.1: 2hr, Steel

11-1.1: 2hr, Steel - Options 2 & 3

12-1.1: 2hr, Steel

[BF] 1505.4 Class C Roof Assemblies

Diagram

Class C roof assemblies are those that are effective against light fire-test exposure. Class C roof assemblies and roof coverings shall be listed and identified as Class C by an approved testing agency.

UpCodes Diagram (1)

Parapets - Exception 5

[BF] 1505.5 Nonclassified Roofing

Nonclassified roofing is approved material that is not listed as a Class A, B or C roof covering assembly or roof covering.

[BF] 1505.6 Fire-Retardant-Treated Wood Shingles and Shakes

Diagram

Fire-retardant-treated wood shakes and shingles are wood shakes and shingles complying with UBC Standard 15-3 or 15-4 which are impregnated by the full-cell vacuum-pressure process with fire-retardant chemicals, and which have been qualified by UBC Standard 15-2 or ASTM E108 or UL 790 for use on Class A, B or C roofs.

Fire-retardant-treated wood shakes and shingles shall comply with ICC-ES EG107 and with the weathering requirements contained in Health and Safety Code Section 13132.7(j). Each bundle shall bear labels from an ICC accredited quality control agency identifying their roof-covering classification and indicating their compliance with ICC-ES EG107 and with the weathering requirements contained in Health and Safety Code Section 13132.7(j).

Health and Safety Code Section 13132.7(j). No wood roof covering materials shall be sold or applied in this state unless both of the following conditions are met:

- The materials have been approved and listed by the State Fire Marshal as complying with the requirements of this section.

- The materials have passed at least five years of the 10-year natural weathering test. The 10-year natural weathering test required by this subdivision shall be conducted in accordance with standard 15-2 of the 1994 edition of the Uniform Building Code at a testing facility recognized by the State Fire Marshal.

UpCodes Diagrams (3)

Wood Shingles - Application

Wood Shingles - Sheathing Requirements

Wood Shingles - Valley Flashing

[BF] 1505.7 Special Purpose Roofs

Special purpose wood shingle or wood shake roofing shall conform to the grading and application requirements of Section 1507.8 or 1507.9. In addition, an underlayment of 5/8-inch (15.9 mm) Type X water-resistant gypsum backing board or gypsum sheathing shall be placed under minimum nominal 1/2-inch-thick (12.7 mm) wood structural panel solid sheathing or 1-inch (25 mm) nominal spaced sheathing.

[BF] 1505.8 Building-Integrated Photovoltaic (BIPV) Systems

BIPV products installed as the roof covering shall be tested, listed and labeled for fire classification in accordance with Section 1505.1.

[BF] 1505.9 Rooftop Mounted Photovoltaic (PV) Panel Systems

Diagram

Rooftop mounted photovoltaic (PV) panel systems shall be tested, listed and identified with a fire classification in accordance with UL 2703. Listed systems shall be installed in accordance with the manufacturer's installation instructions and their listing. The fire classification shall comply with Table 1505.1 based on the type of construction of the building.

UpCodes Diagrams (3)

PV: Setbacks and Pathways on R-3 Occ.

PV: Setbacks and Pathways: Non R-3

Roof Assembly Fire Classification

[BF] 1505.10 Landscaped and Vegetative Roofs

Diagram

Landscaped and vegetative roofs shall comply with Sections 1505.1 and 1507.15. Vegetative roofs shall be installed in accordance with ANSI/SPRI VF-1.

UpCodes Diagram (1)

Rooftop Vegetated Areas

Section 1506 Materials

1506.1 Scope

The requirements set forth in this section shall apply to the application of roof-covering materials specified herein. Roof coverings shall be applied in accordance with this chapter and the roof covering listing as required by Section 1505. Installation of roof coverings shall comply with the applicable provisions of Section 1507.

1506.2 Material Specifications and Physical Characteristics

Roof-covering materials shall conform to the applicable standards listed in this chapter.

1506.3 Product Identification

Roof-covering materials shall be delivered in packages bearing the manufacturer's identifying marks and approved testing agency labels required in accordance with Section 1505. Bulk shipments of materials shall be accompanied with the same information issued in the form of a certificate or on a bill of lading by the manufacturer.

Section 1507 Requirements for Roof Coverings

Diagram

UpCodes Diagrams (14)

Built-Up Roof at Roof-to-Wall Expansion Joint

Example: Built-Up Roofing with Modified Cap Sheet

Example: Liquid-Applied Roofing at Duct Penetration

Example: Liquid-Applied Roofing at Pipe / Conduit Penetrations

Example: Liquid-Applied Roofing at Rooftop Unit Curb Support

Slate Roof: Shingle Layout Plan

Slate Roof: Valley Flashing

Slate Roof: Eave Wood Cant

Slate Roof: Shingle Layout Section

Slate Roof: Ridge/Hip at Flat Roof Intersection

Show 4 more

Building products (3)

FOAMGLAS® T3+ Cellular Glass

Owens Corning

FOAMULAR® & FOAMULAR® NGX® Thermapink® 25 Tapered

Owens Corning

FOAMULAR® NGX® THERMAPINK® 25 XPS

Owens Corning

1507.1 Scope

Roof coverings shall be applied in accordance with the applicable provisions of this section and the manufacturer's installation instructions.

1507.1.1 Underlayment

Diagram

Underlayment in accordance with this section is required for asphalt shingles, clay and concrete tile, metal roof shingles, mineral-surfaced roll roofing, slate and slate-type shingles, wood shingles, wood shakes, metal roof panels and BIPV roof coverings. Such underlayment shall conform to the applicable standards listed in this chapter. Underlayment materials required to comply with ASTM D226, D1970, D2626, D4869, D6380 Class M, D6757 or D8257 shall bear a label indicating compliance with the standard designation and, if applicable, type classification indicated in Table 1507.1.1(1). Underlayment shall be fastened in accordance with Table 1507.1.1(2). Underlayment shall be attached in accordance with Table 1507.1.1(3).

Exception: Structural metal panels that do not require a substrate or underlayment.

TABLE 1507.1.1(1)—UNDERLAYMENT TYPES

| ROOF COVERING | SECTION | MAXIMUM BASIC WIND SPEED, V < 130 MPH IN HURRICANE-PRONE REGIONS OR V < 140 MPH OUTSIDE HURRICANE-PRONE REGIONS | MAXIMUM BASIC WIND SPEED, V ≥ 130 MPH IN HURRICANE-PRONE REGIONS OR V ≥ 140 MPH OUTSIDE HURRICANE-PRONE REGIONS |
| --- | --- | --- | --- |
| Asphalt shingles | 1507.2 | ASTM D226 Type I or II ASTM D1970 ASTM D4869 Type I, II, III or IV ASTM D6757 ASTM D8257 | ASTM D226 Type II ASTM D1970 ASTM D4869 Type III or IV ASTM D8257 |
| Clay and concrete tiles | 1507.3 | ASTM D226 Type II ASTM D1970 ASTM D2626 ASTM D6380 Class M ASTM D8257 | ASTM D226 Type II ASTM D1970 ASTM D8257 |
| Metal roof panels applied to a solid or closely fitted deck | 1507.4 | ASTM D226 Type I or II ASTM D1970 ASTM D4869 Type I, II, III or IV ASTM D8257 | ASTM D226 Type II ASTM D1970 ASTM D4869 Type III or IV ASTM D8257 |
| Metal roof shingles | 1507.5 | ASTM D226 Type I or II ASTM D1970 ASTM D4869 Type I, II, III or IV ASTM D8257 | ASTM D226 Type II ASTM D1970 ASTM D4869 Type III or IV ASTM D8257 |
| Mineral-surfaced roll roofing | 1507.6 | ASTM D226 Type I or II ASTM D1970 ASTM D4869 Type I, II, III or IV ASTM D8257 | ASTM D226 Type II ASTM D1970 ASTM D4869 Type III or IV ASTM D8257 |
| Slate shingles | 1507.7 | ASTM D226 Type II ASTM D1970 ASTM D4869 Type III or IV ASTM D8257 | ASTM D226 Type II ASTM D1970 ASTM D4869 Type III or IV ASTM D8257 |
| Wood shingles | 1507.8 | ASTM D226 Type I or II ASTM D4869 Type I, II, III or IV | ASTM D226 Type II ASTM D4869 Type III or IV |
| Wood shakes applied to a solid sheathing roof deck | 1507.9 | ASTM D226 Type I or II ASTM D4869 Type I, II, III or IV | ASTM D226 Type II ASTM D4869 Type III or IV |
| BIPV roof coverings | 1507.16 | ASTM D226 Type I or II ASTM D1970 ASTM D4869 Type I, II, III or IV ASTM D6757 ASTM D8257 | ASTM D226 Type II ASTM D1970 ASTM D4869 Type III or IV ASTM D8257 |

TABLE 1507.1.1(2)—UNDERLAYMENT APPLICATION

| ROOF COVERING | SECTION | MAXIMUM BASIC WIND SPEED, V < 130 MPH IN HURRICANE-PRONE REGIONS OR V < 140 MPH OUTSIDE HURRICANE-PRONE REGIONS | MAXIMUM BASIC WIND SPEED, V ≥ 130 MPH IN HURRICANE-PRONE REGIONS OR V ≥ 140 MPH OUTSIDE HURRICANE-PRONE REGIONS |
| --- | --- | --- | --- |
| Asphalt shingles | 1507.2 | Underlayment shall be one of the following: - For roof slopes from 2 units vertical in 12 units horizontal (2:12) to 4 units vertical in 12 units horizontal (4:12), underlayment shall be two layers applied in the following manner: Apply a strip of underlayment that is half the width of a full sheet parallel to and starting at the eaves. Starting at the eaves, apply full-width sheets of underlayment, overlapping successive sheets half the width of a full sheet plus 2 inches. End laps shall be 4 inches and shall be offset by 6 feet. Distortions in the underlayment shall not interfere with the ability of the shingles to seal. - For roof slopes of 4 units vertical in 12 units horizontal (4:12) or greater, underlayment shall be one layer applied as follows: Underlayment shall be applied shingle fashion, parallel to and starting from the eaves and lapped 2 inches. Distortions in the underlayment shall not interfere with the ability of the shingles to seal. End laps shall be 4 inches and shall be offset by 6 feet. - A single layer of self-adhering polymer modified bitumen underlayment complying with ASTM D1970, installed in accordance with the underlayment and roof covering manufacturers' installation instructions for the deck material, roof ventilation configuration, and climate exposure of the roof covering. | Underlayment shall be one of the following: - Two layers of mechanically fastened underlayment applied in the following manner: Apply a strip of underlayment felt that is half the width of a full sheet parallel to and starting at the eaves, fastened sufficiently to hold in place. Starting at the eaves, apply full-width sheets of underlayment overlapping successive sheets half the width of a full sheet plus 2 inches. Distortions in the underlayment shall not interfere with the ability of the shingles to seal. End laps shall be 4 inches and shall be offset by 6 feet. - A strip not less than 4 inches in width of self-adhering polymer modified bitumen underlayment complying with ASTM D1970, installed in accordance with the manufacturer's installation instructions for the deck material, shall be applied over all joints in the roof decking. An approved underlayment complying with Table 1507.1.1(1) for the applicable roof covering and basic wind speed shall be applied over the entire roof over the 4-inch-wide membrane strips. Underlayment shall be applied in accordance with this table using the application requirements for where the maximum basic wind speed is less than 130 mph. - A single layer of self-adhering polymer modified bitumen underlayment complying with ASTM D1970, installed in accordance with the underlayment and roof covering manufacurers' installation instructions for the deck material, roof ventilation configuration and climate exposure of the roof covering. |
| Clay and concrete tile | 1507.3 | Underlayment shall be one of the following: - For roof slopes from 2 1/2 units vertical in 12 units horizontal (2 1/2:12) to 4 units vertical in 12 units horizontal (4:12), underlayment shall be not fewer than two layers applied in the following manner: Apply a strip of underlayment that is half the width of a full sheet parallel to and starting at the eaves. Starting at the eaves, a full-width strip of underlayment felt shall be applied, overlapping successive sheets half the width of a full sheet plus 2 inches. End laps shall be 4 inches and shall be offset by 6 feet. - For roof slopes of 4 units vertical in 12 units horizontal (4:12) or greater, underlayment shall be one layer applied as follows: Underlayment shall be applied shingle fashion, parallel to and starting from the eaves and lapped 2 inches. End laps shall be 4 inches and shall be offset by 6 feet. - A single layer of self-adhering polymer modified bitumen underlayment complying with ASTM D1970, installed in accordance with the underlayment and roof covering manufacturers' installation instructions for the deck material, roof ventilation configuration, and climate exposure of the roof covering. | Underlayment shall be one of the following: - Two layers of mechanically fastened underlayment applied in the following manner: Apply a strip of underlayment that is half the width of a full sheet parallel to and starting at the eaves, fastened sufficiently to hold in place. Starting at the eaves, apply full-width sheets of underlayment, overlapping successive sheets half the width of a full sheet plus 2 inches. Distortions in the underlayment shall not interfere with the ability of the shingles to seal. End laps shall be 4 inches and shall be offset by 6 feet. - A strip not less than 4 inches in width of self-adhering polymer modified bitumen underlayment complying with ASTM D1970, installed in accordance with the manufacturer's installation instructions for the deck material, shall be applied over all joints in the roof decking. An approved underlayment complying with Table 1507.1.1(1) for the applicable roof covering and basic wind speed shall be applied over the entire roof over the 4-inch-wide membrane strips. Underlayment shall be applied in accordance with this table using the application requirements for where the maximum basic wind speed is less than 130 mph. - A single layer of self-adhering polymer modified bitumen underlayment complying with ASTM D1970, installed in accordance with the underlayment and roof covering manufacurers' installation instructions for the deck material, roof ventilation configuration, and climate exposure of the roof covering. |
| Metal roof panels | 1507.4 | Apply in accordance with the manufacturer's installation instructions | Underlayment shall be one of the following: - Two layers of mechanically fastened underlayment applied in the following manner: Apply a strip of underlayment that is half the width of a full sheet parallel to and starting at the eaves, fastened sufficiently to hold in place. Starting at the eaves, apply full-width sheets of underlayment, overlapping successful sheets half the width of a full sheet plus 2 inches. Distortions in the underlayment shall not interfere with the ability of the shingles to seal. End laps shall be 4 inches and shall be offset by 6 feet. - A strip not less than 4 inches in width of self-adhering polymer modified bitumen underlaymentcomplying with ASTM D1970, installed in accordance with the manufacturer's installation instructions for the deck material, shall be applied over all joints in the roof decking. An approved underlayment complying with Table 1507.1.1(1) for the applicable roof covering and basic wind speed shall be applied over the entire roof over the 4-inch-wide membrane strips. Underlayment shall be applied in accordance with this table using the application requirements for where the maximum basic wind speed is less than 130 mph. - A single layer of self-adhering polymer modified bitumen underlayment complying with ASTM D1970, installed in accordance with the underlayment and roof covering manufacurers' installation instructions for the deck material, roof ventilation configuration, and climate exposure of the roof covering. |
| Metal roof shingles | 1507.5 |  |  |
| Mineral-surfaced roll roofing | 1507.6 |  |  |
| Slate shingles | 1507.7 |  |  |
| Wood shingles | 1507.8 |  |  |
| Wood shakes | 1507.9 |  |  |
| BIPV roof coverings | 1507.16 | Underlayment shall be one of the following: - For roof slopes from 3 units vertical in 12 units horizontal (3:12) to 4 units vertical in 12 units horizontal (4:12), underlayment shall be two layers applied in the following manner: Apply a strip of underlayment that is half the width of a full sheet parallel to and starting at the eaves. Starting at the eaves, apply full-width sheets of underlayment, overlapping successive sheets half the width of a full sheet plus 2 inches. End laps shall be 4 inches and shall be offset by 6 feet. Distortions in the underlayment shall not interfere with the ability of the shingles to seal. - For roof slopes of 4 units vertical in 12 units horizontal (4:12) or greater, underlayment shall be one layer applied as follows: Underlayment shall be applied shingle fashion, parallel to and starting from the eaves and lapped 2 inches. Distortions in the underlayment shall not interfere with the ability of the shingles to seal. End laps shall be 4 inches and shall be offset by 6 feet. - A single layer of self-adhering polymer modified bitumen underlayment complying with ASTM D1970, installed in accordance with the underlayment and roof covering manufacturers' installation instructions for the deck material, roof ventilation configuration, and climate exposure of the roof covering. | Underlayment shall be one of the following: - Two layers of mechanically fastened underlayment applied in the following manner: Apply a strip of underlayment that is half the width of a full sheet parallel to and starting at the eaves, fastened sufficiently to hold in place. Starting at the eaves, apply full-width sheets of underlayment, overlapping successive sheets half the width of a full sheet plus 2 inches. Distortions in the underlayment shall not interfere with the ability of the shingles to seal. End laps shall be 4 inches and shall be offset by 6 feet. - A strip not less than 4 inches in width of self-adhering polymer modified bitumen underlayment complying with ASTM D1970, installed in accordance with the manufacturer's installation instructions for the deck material, shall be applied over all joints in the roof decking. An approved underlayment complying withTable 1507.1.1(1) for the applicable roof covering and basic wind speed shall be applied over the entire roof over the 4-inch-wide membrane strips. Underlayment shall be applied in accordance with this table using the application requirements for where the maximum basic wind speed is less than 130 mph. - A single layer of self-adhering polymer modified bitumen underlayment complying with ASTM D1970, installed in accordance with the underlayment and roof covering manufacturers' installation instructions for the deck material, roof ventilation configuration, and climate exposure of the roof covering. |

For SI: 1 inch = 25.4 mm, 1 foot = 304.8 mm, 1 mile per hour = 0.447 m/s.

TABLE 1507.1.1(3)—UNDERLAYMENT FASTENING

| ROOF COVERING | SECTION | MAXIMUM BASIC WIND SPEED, V < 130 MPH IN HURRICANE-PRONE REGIONS OR V < 140 MPH OUTSIDE HURRICANE-PRONE REGIONS | MAXIMUM BASIC WIND SPEED, V ≥ 130 MPH IN HURRICANE-PRONE REGIONS OR V ≥ 140 MPH OUTSIDE HURRICANE-PRONE REGIONS |
| --- | --- | --- | --- |
| Asphalt shingles | 1507.2 | Fastened sufficiently to hold in place | Mechanically fastened underlayment shall be fastened with corrosion-resistant fasteners in a grid pattern of not greater than 12 inches horizontally and vertically between side laps with a 6-inch spacing at side and end laps. Mechanically fastened underlayment shall be fastened using annular ring or deformed shank nails with 1-inch diameter metal or plastic caps. Metal caps shall have a thickness of not less than 32-gage (0.0134 inch) sheet metal. Power-driven metal caps shall have a minimum thickness of 0.010 inch. Minimum thickness of the outside edge of plastic caps shall be 0.035 inch. The cap nail shank shall be not less than 0.083 inch. The cap nail shank shall have a length sufficient to penetrate through the roof sheathing or not less than 3/4 inch into the roof sheathing. Self-adhering polymer modified bitumen underlayment shall be installed in accordance with the underlayment and roof covering manufacturers' installation instructions for the deck material, roof ventilation configuration and climate exposure of the roof covering . |
| Clay and concrete tile | 1507.3 |  |  |
| BIPV roof coverings | 1507.16 |  |  |
| Metal roof panels | 1507.4 | Manufacturer's installation instructions | Mechanically fastened underlayment shall be fastened with corrosion-resistant fasteners in a grid pattern of not greater than 12 inches horizontally and vertically between side laps with a 6-inch spacing at side and end laps. Mechanically fastened underlayment shall be fastened using annular ring or deformed shank nails with 1-inch diameter metal or plastic caps. Metal caps shall have a thickness of not less than 32-gage sheet metal. Power-driven metal caps shall have a minimum thickness of 0.010 inch. Minimum thickness of the outside edge of plastic caps shall be 0.035 inch. The cap nail shank shall be not less than 0.083 inch. The cap nail shank shall have a length sufficient to penetrate through the roof sheathing or not less than 3/4 inch into the roof sheathing. Self-adhering polymer modified bitumen underlayment shall be installed in accordance with the underlayment and roof covering manufacturers' installation instructions for the deck material, roof ventilation configuration and climate exposure of the roof covering. |
| Metal roof shingles | 1507.5 |  |  |
| Mineral-surfaced roll roofing | 1507.6 |  |  |
| Slate shingles | 1507.7 |  |  |
| Wood shingles | 1507.8 |  |  |
| Wood shakes | 1507.9 |  |  |

For SI: 1 inch = 25.4 mm, 1 mile per hour = 0.447 m/s.

UpCodes Diagrams (3)

Asphalt Shingles and Fasteners

Slate Roof: Shingle Layout Plan

Slate Roof: Vertical Wall Flashing

1507.1.2 Ice Barriers

Diagram

In areas where there has been a history of ice forming along the eaves causing a backup of water, an ice barrier shall be installed for asphalt shingles, metal roof shingles, mineral-surfaced roll roofing, slate and slate-type shingles, wood shingles, and wood shakes. The ice barrier shall consist of not less than two layers of underlayment cemented together, or a self-adhering polymer modified bitumen sheet shall be used in place of normal underlayment and extend from the lowest edges of all roof surfaces to a point not less than 24 inches (610 mm) inside the exterior wall line of the building.

Exception: Detached accessory structures that do not contain conditioned floor area.

UpCodes Diagram (1)

Primary

Ice Barriers: Roofing

1507.2 Asphalt Shingles

Diagram

The installation of asphalt shingles shall comply with the provisions of this section.

UpCodes Diagrams (3)

Proper Location for Asphalt Shingle Fasteners

Asphalt Shingles and Fasteners

Asphalt Shingles: Valley Linings

1507.2.1 Deck Requirements

Asphalt shingles shall be fastened to solidly sheathed decks.

1507.2.2 Slope

Asphalt shingles shall only be used on roof slopes of 2 units vertical in 12 units horizontal (17-percent slope) or greater. For roof slopes from 2 units vertical in 12 units horizontal (17-percent slope) up to 4 units vertical in 12 units horizontal (33-percent slope), double underlayment application is required in accordance with Section 1507.1.1.

1507.2.3 Underlayment

Underlayment shall comply with Section 1507.1.1.

1507.2.4 Asphalt Shingles

Asphalt shingles shall comply with ASTM D3462.

1507.2.5 Fasteners

Diagram

Fasteners for asphalt shingles shall be galvanized, stainless steel, aluminum or copper roofing nails, minimum 12-gage [0.105 inch (2.67 mm)] shank with a minimum 3/8-inch-diameter (9.5 mm) head, of a length to penetrate through the roofing materials and not less than 3/4 inch (19.1 mm) into the roof sheathing. Where the roof sheathing is less than 3/4 inch (19.1 mm) thick, the nails shall penetrate through the sheathing. Fasteners shall comply with ASTM F1667.

UpCodes Diagrams (2)

Primary

Asphalt Shingles and Fasteners

Primary

Proper Location for Asphalt Shingle Fasteners

1507.2.6 Attachment

Asphalt shingles shall have the minimum number of fasteners required by the manufacturer, but not less than four fasteners per strip shingle or two fasteners per individual shingle. Where the roof slope exceeds 21 units vertical in 12 units horizontal (21:12), shingles shall be installed as required by the manufacturer.

1507.2.7 Ice Barrier

Diagram

Where required, ice barriers shall comply with Section 1507.1.2.

UpCodes Diagram (1)

Ice Barriers: Roofing

1507.2.8 Flashings

Diagram

Flashing for asphalt shingles shall comply with this section. Flashing shall be applied in accordance with this section and the asphalt shingle manufacturer's instructions.

UpCodes Diagram (1)

Drip Edge for Asphalt Shingles

1507.2.8.1 Base and Cap Flashing

Base and cap flashing shall be installed in accordance with the manufacturer's instructions. Base flashing shall be of either corrosion-resistant metal of minimum nominal 0.019-inch (0.483 mm) thickness or mineral-surfaced roll roofing weighing not less than 77 pounds per 100 square feet (3.76 kg/m2). Cap flashing shall be corrosion-resistant metal of minimum nominal 0.019-inch (0.483 mm) thickness.

1507.2.8.2 Valleys

Diagram

Valley linings shall be installed in accordance with the manufacturer's instructions before applying shingles. Valley linings of the following types shall be permitted:

- For open valleys (valley lining exposed) lined with metal, the valley lining shall be not less than 24 inches (610 mm) wide and of any of the corrosion-resistant metals in Table 1507.2.8.2.

- For open valleys, valley lining of two plies of mineral-surfaced roll roofing complying with ASTM D3909 or ASTM D6380 shall be permitted. The bottom layer shall be 18 inches (457 mm) and the top layer not less than 36 inches (914 mm) wide.

- For closed valleys (valleys covered with shingles), valley lining of one ply of smooth roll roofing complying with ASTM D6380 and not less than 36 inches (914 mm) wide or types as described in Item 1 or 2 above shall be permitted. Self-adhering polymer modified bitumen underlayment bearing a label indicating compliance with ASTM D1970 and not less than 36 inches (914 mm) wide shall be permitted in lieu of the lining material.

TABLE 1507.2.8.2—VALLEY LINING MATERIAL

| MATERIAL | MINIMUM THICKNESS | GAGE | WEIGHT |
| --- | --- | --- | --- |
| Aluminum | 0.024 in. | — | — |
| Cold-rolled copper | 0.0216 in. | — | ASTM B370, 16 oz. per square ft. |
| Copper | — | — | 16 oz. |
| Galvanized steel | 0.0179 in. | 26 (zinc-coated G90) | — |
| High-yield copper | 0.0162 in. | — | ASTM B370, 12 oz. per square ft. |
| Lead | — | — | 2.5 pounds |
| Lead-coated copper | 0.0216 in. | — | ASTM B101, 16 oz. per square ft. |
| Lead-coated high-yield copper | 0.0162 in. | — | ASTM B101, 12 oz. per square ft. |
| Painted terne | — | — | 20 pounds |
| Stainless steel | — | 28 | — |
| Zinc alloy | 0.027 in. | — | — |

For SI: 1 inch = 25.4 mm, 1 pound = 0.454 kg, 1 ounce = 28.35 g, 1 square foot = 0.0929 m2.

UpCodes Diagram (1)

Primary

Asphalt Shingles: Valley Linings

1507.2.8.3 Drip Edge

Diagram

A drip edge shall be provided at eaves and rake edges of shingle roofs. Adjacent segments of the drip edge shall be lapped not less than 2 inches (51 mm). The vertical leg of drip edges shall be not less than 11/2 inches (38 mm) in width and shall extend not less than 1/4 inch (6.4 mm) below sheathing. The drip edge shall extend back on the roof not less than 2 inches (51 mm). Underlayment shall be installed over drip edges along eaves. Drip edges shall be installed over underlayment along rake edges. Drip edges shall be mechanically fastened at intervals not greater than 12 inches (305 mm) on center.

UpCodes Diagram (1)

Primary

Drip Edge for Asphalt Shingles

1507.3 Clay and Concrete Tile

Diagram

The installation of clay and concrete tile shall comply with the provisions of this section.

UpCodes Diagram (1)

Roof Tile Valley Flashing

1507.3.1 Deck Requirements

Concrete and clay tile shall be installed only over solid sheathing.

Exception: Spaced lumber sheathing shall be permitted in Seismic Design Categories A, B and C.

1507.3.2 Deck Slope

Clay and concrete roof tile shall be installed on roof slopes of 21/2 units vertical in 12 units horizontal (21-percent slope) or greater. For roof slopes from 21/2 units vertical in 12 units horizontal (21-percent slope) to 4 units vertical in 12 units horizontal (33-percent slope), double underlayment application is required in accordance with Section 1507.1.1.

1507.3.3 Underlayment

Unless otherwise noted, required underlayment shall conform to: ASTM D226, Type II; ASTM D2626 or ASTM D6380, Class M mineral-surfaced roll roofing.

1507.3.4 Clay Tile

Clay roof tile shall comply with ASTM C1167.

1507.3.5 Concrete Tile

Concrete roof tile shall comply with ASTM C1492.

1507.3.6 Fasteners

Tile fasteners shall be corrosion resistant and not less than 11-gage, [0.120 inch (3 mm)], 5/16-inch (8.0 mm) head, and of sufficient length to penetrate the deck not less than 3/4 inch (19.1 mm) or through the thickness of the deck, which-ever is less. Attaching wire for clay or concrete tile shall not be smaller than 0.083 inch (2.1 mm). Perimeter fastening areas include three tile courses but not less than 36 inches (914 mm) from either side of hips or ridges and edges of eaves and gable rakes.

1507.3.7 Attachment

Clay and concrete roof tiles shall be fastened in accordance with Table 1507.3.7.

TABLE 1507.3.7—CLAY AND CONCRETE TILE ATTACHMENTa, b, c

| GENERAL—CLAY OR CONCRETE ROOF TILE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Maximum Allowable Stress Design Wind Speed, Vasd f (mph) | Mean roof height (feet) | Roof slope < 3:12 | Roof slope 3:12 and over |  |
| 85 | 0-60 | One fastener per tile. Flat tile without vertical laps, two fasteners per tile. | Two fasteners per tile. Only one fastener on slopes of 7:12 and less for tiles with installed weight exceeding 7.5 lbs./sq. ft. having a width not more than 16 inches. |  |
| 100 | 0-40 |  |  |  |
| 100 | > 40-60 | The head of all tiles shall be nailed. The nose of all eave tiles shall be fastened with approved clips. Rake tiles shall be nailed with two nails. The nose of all ridge, hip and rake tiles shall be set in a bead of roofer's mastic. |  |  |
| 110 | 0-60 | The fastening system shall resist the wind forces in Section 1609.6.3.1. |  |  |
| 120 | 0-60 | The fastening system shall resist the wind forces in Section 1609.6.3.1. |  |  |
| 130 | 0-60 | The fastening system shall resist the wind forces in Section 1609.6.3.1. |  |  |
| All | > 60 | The fastening system shall resist the wind forces in Section 1609.6.3.1. |  |  |
| INTERLOCKING CLAY OR CONCRETE ROOF TILE WITH PROJECTING ANCHOR LUGSd, e (Installations on spaced/solid sheathing with battens or spaced sheathing) |  |  |  |  |
| Maximum Allowable Stress Design Wind Speed, Vasdf (mph) | Mean roof height (feet) | Roof slope < 5:12 | Roof slope 5:12 < 12:12 | Roof slope 12:12 and over |
| 85 | 0-60 | Fasteners are not required. Tiles with installed weight less than 9 lbs./sq. ft. require not fewer than one fastener per tile. | One fastener per tile every other row. Perimeter tiles require one fastener. Tiles with installed weight less than 9 lbs./sq. ft. require not fewer than one fastener per tile. | One fastener required for every tile. Tiles with installed weight less than 9 lbs./sq. ft. require not fewer than one fastener per tile. |
| 100 | 0-40 |  |  |  |
| 100 | > 40-60 | The head of all tiles shall be nailed. The nose of all eave tiles shall be fastened with approved clips. Rake5 tiles shall be nailed with two nails. The nose of all ridge, hip and rake tiles shall be set in a bead of roofer's mastic. |  |  |
| 110 | 0-60 | The fastening system shall resist the wind forces in Section 1609.6.3.1. |  |  |
| 120 | 0-60 | The fastening system shall resist the wind forces in Section 1609.6.3.1. |  |  |
| 130 | 0-60 | The fastening system shall resist the wind forces in Section 1609.6.3.1. |  |  |
| All | > 60 | The fastening system shall resist the wind forces in Section 1609.6.3.1. |  |  |
| INTERLOCKING CLAY OR CONCRETE ROOF TILE WITH PROJECTING ANCHOR LUGS (Installations on solid sheathing without battens) |  |  |  |  |
| Maximum Allowable Stress Wind Speed, Vasdf (mph) | Mean roof height (feet) | All roof slopes |  |  |
| 85 | 0-60 | One fastener per tile. |  |  |
| 100 | 0-40 | One fastener per tile. |  |  |
| 100 | > 40-60 | The head of all tiles shall be nailed. The nose of all eave tiles shall be fastened with approved clips. Rake tiles shall be nailed with two nails. The nose of all ridge, hip and rake tiles shall be set in a bead of roofer's mastic. |  |  |
| 110 | 0-60 | The fastening system shall resist the wind forces in Section 1609.6.3.1. |  |  |
| 120 | 0-60 | The fastening system shall resist the wind forces in Section 1609.6.3.1. |  |  |
| 130 | 0-60 | The fastening system shall resist the wind forces in Section 1609.6.3.1. |  |  |
| All | > 60 | The fastening system shall resist the wind forces in Section 1609.6.3.1. |  |  |

For SI: 1 inch = 25.4 mm, 1 foot = 304.8 mm, 1 mile per hour = 0.447 m/s, 1 pound per square foot = 4.882 kg/m2.

- Minimum fastener size. Corrosion-resistant nails not less than No. 11 gage with 5/16-inch head. Fasteners shall be long enough to penetrate into the sheathing 3/4 inch or through the thickness of the sheathing, whichever is less. Attaching wire for clay and concrete tile shall not be smaller than 0.083 inch.

- Snow areas. Not fewer than two fasteners per tile are required or battens and one fastener.

- Roof slopes greater than 24:12. The nose of all tiles shall be securely fastened.

- Horizontal battens. Battens shall be not less than 1 inch by 2 inches nominal. Provisions shall be made for drainage by a riser of not less than 1/8 inch at each nail or by 4-foot-long battens with not less than a 1/2-inch separation between battens. Horizontal battens are required for slopes over 7:12.

- Perimeter fastening areas include three tile courses but not less than 36 inches from either side of hips or ridges and edges of eaves and gable rakes.

- Vasd shall be determined in accordance with Section 1609.3.1.

1507.3.8 Application

Tile shall be applied according to the manufacturer's installation instructions, based on the following:

- Climatic conditions.

- Roof slope.

- Underlayment system.

- Type of tile being installed.

1507.3.9 Flashing

Diagram

At the juncture of the roof vertical surfaces, flashing and counterflashing shall be provided in accordance with the manufacturer's installation instructions, and where of metal, shall be not less than 0.019-inch (0.48 mm) (No. 26 galvanized sheet gage) corrosion-resistant metal. The valley flashing shall extend not less than 11 inches (279 mm) from the centerline each way and have a splash diverter rib not less than 1 inch (25 mm) high at the flow line formed as part of the flashing. Sections of flashing shall have an end lap of not less than 4 inches (102 mm). For roof slopes of three units vertical in 12 units horizontal (25-percent slope) and over, the valley flashing shall have a 36-inch-wide (914 mm) underlayment of either one layer of Type I underlayment running the full length of the valley, or a self-adhering polymer-modified bitumen sheet bearing a label indicating compliance with ASTM D1970, in addition to other required underlayment. In areas where the average daily temperature in January is 25°F (-4°C) or less or where there is a possibility of ice forming along the eaves causing a backup of water, the metal valley flashing underlayment shall be solid cemented to the roofing underlayment for slopes under seven units vertical in 12 units horizontal (58-percent slope) or self-adhering polymer-modified bitumen sheet shall be installed.

UpCodes Diagram (1)

Primary

Roof Tile Valley Flashing

1507.3.10 Additional Requirements

[DSA-SS & DSA-SS/CC, OSHPD 1, 1R, 2, 4 & 5] In addition to the requirements of 1507.3.6 and 1507.3.7, the installation of clay and concrete tile roof coverings shall comply with seismic anchorage provisions of Section 1513.

1507.4 Metal Roof Panels

The installation of metal roof panels shall comply with the provisions of this section.

1507.4.1 Deck Requirements

Metal roof panel roof coverings shall be applied to a solid or closely fitted deck, except where the roof covering is specifically designed to be applied to spaced supports.

1507.4.2 Deck Slope

Minimum slopes for metal roof panels shall comply with the following:

- The minimum slope for lapped, nonsoldered seam metal roof panels without applied lap sealant shall be three units vertical in 12 units horizontal (25-percent slope).

- The minimum slope for lapped, nonsoldered seam metal roof panels with applied lap sealant shall be one-half unit vertical in 12 units horizontal (4-percent slope). Lap sealants shall be applied in accordance with the approved manufacturer's installation instructions.

- The minimum slope for standing-seam metal roof panel systems shall be one-quarter unit vertical in 12 units horizontal (2-percent slope).

1507.4.3 Material Standards

Metal-sheet roof covering systems that incorporate supporting structural members shall be designed in accordance with Chapter 22. Metal-sheet roof coverings shall comply with Table 1507.4.3.

TABLE 1507.4.3—METAL ROOF COVERINGS

| ROOF COVERING TYPE | STANDARD APPLICATION RATE/THICKNESS |
| --- | --- |
| 5% aluminum alloy-coated steel | ASTM A875, GF60 |
| Aluminum | ASTM B209, 0.024 inch minimum thickness for roll-formed panels and 0.019 inch minimum thickness for press-formed shingles. |
| Aluminum-coated steel | ASTM A463, T2 65 |
| 55% aluminum-zinc alloy coated steel | ASTM A792 AZ 50 |
| Cold-rolled copper | ASTM B370 minimum 16 oz./sq. ft. and 12 oz./sq. ft. high yield copper for metal-sheet roof covering systems: 12 oz./sq. ft. for preformed metal shingle systems. |
| Copper | 16 oz./sq. ft. for metal-sheet roof-covering systems; 12 oz./sq. ft. for preformed metal shingle systems. |
| Galvanized steel | ASTM A653 G-90 zinc-coated.a |
| Hard lead | 2 lbs./sq. ft. |
| Lead-coated copper | ASTM B101 |
| Prepainted steel | ASTM A755 |
| Soft lead | 3 lbs./sq. ft. |
| Stainless steel | ASTM A240, 300 Series Alloys |
| Steel | ASTM A924 |
| Terne and terne- coated stainless | Terne coating of 40 lbs. per double base box, field painted where applicable in accordance with manufacturer's installation instructions. |
| Zinc | 0.027 inch minimum thickness; 99.995% electrolytic high-grade zinc with alloy addi tives of copper (0.08% - 0.20%), titanium (0.07% - 0.12%) and aluminum (0.015%). |

For SI: 1 ounce per square foot = 0.305 kg/m2, 1 pound per square foot = 4.882 kg/m2, 1 inch = 25.4 mm, 1 pound = 0.454 kg.

- For Group U buildings, the minimum coating thickness for ASTM A653 galvanized steel roofing shall be G60.

1507.4.4 Attachment

Metal roof panels shall be secured to the supports in accordance with the approved manufacturer's fasteners. In the absence of manufacturer recommendations, the following fasteners shall be used:

- Galvanized fasteners shall be used for steel roofs.

- Copper, brass, bronze, copper alloy or 300 series stainless-steel fasteners shall be used for copper roofs.

- Stainless-steel fasteners are acceptable for all types of metal roofs.

- Aluminum fasteners are acceptable for aluminum roofs attached to aluminum supports.

1507.4.5 Underlayment and High Wind

Underlayment shall comply with Section 1507.1.1.

1507.5 Metal Roof Shingles

The installation of metal roof shingles shall comply with the provisions of this section.

1507.5.1 Deck Requirements

Metal roof shingles shall be applied to a solid or closely fitted deck, except where the roof covering is specifically designed to be applied to spaced sheathing.

1507.5.2 Deck Slope

Metal roof shingles shall not be installed on roof slopes below 3 units vertical in 12 units horizontal (25-percent slope).

1507.5.3 Underlayment

Underlayment shall comply with Section 1507.1.1.

1507.5.4 Ice Barrier

Diagram

Where required, ice barriers shall comply with Section 1507.1.2.

UpCodes Diagram (1)

Ice Barriers: Roofing

1507.5.5 Material Standards

Metal roof shingle roof coverings shall comply with Table 1507.4.3.

1507.5.6 Attachment

Metal roof shingles shall be secured to the roof in accordance with the approved manufacturer's installation instructions.

1507.5.7 Flashing

Roof valley flashing shall be of corrosion-resistant metal of the same material as the roof covering or shall comply with the standards in Table 1507.4.3. The valley flashing shall extend not less than 8 inches (203 mm) from the centerline each way and shall have a splash diverter rib not less than 3/4 inch (19.1 mm) high at the flow line formed as part of the flashing. Sections of flashing shall have an end lap of not less than 4 inches (102 mm). In areas where the average daily temperature in January is 25°F (-4°C) or less or where there is a possibility of ice forming along the eaves causing a backup of water, the metal valley flashing shall have a 36-inch-wide (914 mm) underlayment directly under it consisting of either one layer of underlayment running the full length of the valley or a self-adhering polymer-modified bitumen sheet bearing a label indicating compliance with ASTM D1970, in addition to underlayment required for metal roof shingles. The metal valley flashing underlayment shall be solidly cemented to the roofing underlayment for roof slopes under seven units vertical in 12 units horizontal (58-percent slope) or self-adhering polymer-modified bitumen sheet shall be installed.

1507.6 Mineral-Surfaced Roll Roofing

The installation of mineral-surfaced roll roofing shall comply with this section.

1507.6.1 Deck Requirements

Mineral-surfaced roll roofing shall be fastened to solidly sheathed roofs.

1507.6.2 Deck Slope

Mineral-surfaced roll roofing shall not be applied on roof slopes below 1 unit vertical in 12 units horizontal (8-percent slope).

1507.6.3 Underlayment

Underlayment shall comply with Section 1507.1.1.

1507.6.4 Ice Barrier

Where required, ice barriers shall comply with Section 1507.1.2.

1507.6.5 Material Standards

Mineral-surfaced roll roofing shall conform to ASTM D3909 or ASTM D6380.

1507.7 Slate Shingles

Diagram

The installation of slate shingles shall comply with the provisions of this section.

UpCodes Diagrams (7)

Primary

Slate Roof: Ridge/Hip at Flat Roof Intersection

Primary

Slate Roof: Shingle Layout Section

Primary

Slate Roof: Eave Wood Cant

Primary

Slate Roof: Valley Flashing

Primary

Slate Roof: Shingle Layout Plan

Primary

Slate Roofing

Slate Roof: Vertical Wall Flashing

1507.7.1 Deck Requirements

Slate shingles shall be fastened to solidly sheathed roofs.

1507.7.2 Deck Slope

Slate shingles shall only be used on slopes of 4 units vertical in 12 units horizontal (4:12) or greater.

1507.7.3 Underlayment

Underlayment shall comply with Section 1507.1.1.

1507.7.4 Ice Barrier

Diagram

Where required, ice barriers shall comply with Section 1507.1.2.

UpCodes Diagram (1)

Ice Barriers: Roofing

1507.7.5 Material Standards

Slate shingles shall comply with ASTM C406.

1507.7.6 Application

Diagram

Minimum headlap for slate shingles shall be in accordance with Table 1507.7.6. Slate shingles shall be secured to the roof with two fasteners per slate.

TABLE 1507.7.6—SLATE SHINGLE HEADLAP

| SLOPE | HEADLAP (inches) |
| --- | --- |
| 4:12 < slope < 8:12 | 4 |
| 8:12 < slope < 20:12 | 3 |
| slope ≥ 20:12 | 2 |

For SI: 1 inch = 25.4 mm.

UpCodes Diagrams (5)

Primary

Slate Roof: Ridge/Hip at Flat Roof Intersection

Primary

Slate Roof: Shingle Layout Section

Primary

Slate Roof: Eave Wood Cant

Primary

Slate Roof: Shingle Layout Plan

Primary

Slate Roofing

1507.7.7 Flashing

Diagram

Flashing and counterflashing shall be made with sheet metal. Valley flashing shall be not less than 15 inches (381 mm) wide. Valley and flashing metal shall be a minimum uncoated thickness of 0.0179-inch (0.455 mm) zinc-coated G90. Chimneys, stucco or brick walls shall have not fewer than two plies of felt for a cap flashing consisting of a 4-inch-wide (102 mm) strip of felt set in plastic cement and extending 1 inch (25 mm) above the first felt and a top coating of plastic cement. The felt shall extend over the base flashing 2 inches (51 mm).

UpCodes Diagrams (2)

Primary

Slate Roof: Vertical Wall Flashing

Primary

Slate Roof: Valley Flashing

1507.7.8 Additional Requirements

[DSA-SS & DSA-SS/CC, OSHPD 1, 1R, 2, 4 & 5] In addition to the requirements of Section 1507.7.5, the installation of slate shingle roof coverings shall comply with the requirements of Sections 1507.3.6 and 1507.3.7, and the seismic anchorage provisions of Section 1513.

1507.8 Wood Shingles

Diagram

The installation of wood shingles shall comply with the provisions of this section and Table 1507.8.

TABLE 1507.8—WOOD SHINGLE AND SHAKE INSTALLATION

| ROOF ITEM | WOOD SHINGLES | WOOD SHAKES |
| --- | --- | --- |
| 1. Roof slope | Wood shingles shall be installed on slopes of not less than 3 units vertical in 12 units horizontal (3:12). | Wood shakes shall be installed on slopes of not less than 4 units vertical in 12 units horizontal (4:12). |
| 2. Deck requirement |  |  |
| Temperate climate | Shingles shall be applied to roofs with solid or spaced sheathing. Where spaced sheathing is used, sheathing boards shall be not less than 1" × 4" nominal dimensions and shall be spaced on centers equal to the weather exposure to coincide with the placement of fasteners. | Shakes shall be applied to roofs with solid or spaced sheathing. Where spaced sheathing is used, sheathing boards shall be not less than 1" × 4" nominal dimensions and shall be spaced on centers equal to the weather exposure to coincide with the placement of fasteners. Where 1" × 4" spaced sheathing is installed at 10 inches, boards must be installed between the sheathing boards. |
| In areas where the average daily temperature in January is 25°F or less or where there is a possibility of ice forming along the eaves causing a backup of water. | Solid sheathing is required. | Solid sheathing is required. |
| 3. Interlayment | No requirements. | Interlayment shall comply with ASTM D226, Type 1. |
| 4. Underlayment |  |  |
| Temperate climate | Underlayment shall comply with Section 1507.1.1. | Underlayment shall comply with Section 1507.1.1. |
| 5. Application |  |  |
| Attachment | Fasteners for wood shingles shall be hot-dipped galvanized or Type 304 (Type 316 for coastal areas) stainless steel with a minimum penetration of 0.75 inch into the sheathing. For sheathing less than 0.5 inch thick, the fasteners shall extend through the sheathing. | Fasteners for wood shakes shall be hot-dipped galvanized or Type 304 (Type 316 for coastal areas) with a minimum penetration of 0.75 inch into the sheathing. For sheathing less than 0.5 inch thick, the fasteners shall extend through the sheathing. |
| No. of fasteners | Two per shingle. | Two per shake. |
| Exposure | Weather exposures shall not exceed those set forth in Table 1507.8.7. | Weather exposures shall not exceed those set forth in Table 1507.9.8. |
| Method | Shingles shall be laid with a side lap of not less than 1.5 inches between joints in courses, and no two joints in any three adjacent courses shall be in direct alignment. Spacing between shingles shall be 0.25 to 0.375 inch. | Shakes shall be laid with a side lap of not less than 1.5 inches between joints in adjacent courses. Spacing between shakes shall not be less than 0.375 inch or more than 0.625 inch for shakes and taper sawn shakes of naturally durable wood and shall be 0.25 to 0.375 inch for preservative-treated taper sawn shakes. |
| Flashing | In accordance with Section 1507.8.8. | In accordance with Section 1507.9.9. |

For SI: 1 inch = 25.4 mm, °C = [(°F) - 32]/1.8.

UpCodes Diagrams (3)

Wood Shingles - Application

Wood Shingles - Sheathing Requirements

Wood Shingles - Valley Flashing

1507.8.1 Deck Requirements

Wood shingles shall be installed on solid or spaced sheathing. Where spaced sheathing is used, sheathing boards shall be not less than 1-inch by 4-inch (25 mm by 102 mm) nominal dimensions and shall be spaced on centers equal to the weather exposure to coincide with the placement of fasteners. Where 1-inch × 4-inch (25 mm × 102 mm) spaced sheathing is installed at 10 inches (254 mm) on center or greater, additional 1-inch × 4-inch (25 mm × 102 mm) boards shall be installed between the sheathing boards. When wood shingles are installed over spaced sheathing and the underside of the shingles are exposed to the attic space, the attic shall be ventilated in accordance with Section 1202.2. The shingles shall not be backed with materials that will occupy the required air gap space and prevent the free movement of air on the interior side of the spaced sheathing.

1507.8.1.1 Solid Sheathing Required

Solid sheathing is required in areas where the average daily temperature in January is 25°F (-4°C) or less or where there is a possibility of ice forming along the eaves causing a backup of water.

1507.8.2 Deck Slope

Wood shingles shall be installed on slopes of not less than 3 units vertical in 12 units horizontal (25-percent slope).

1507.8.3 Underlayment

Underlayment shall comply with Section 1507.1.1.

1507.8.4 Ice Barrier

Where required, ice barriers shall comply with Section 1507.1.2.

1507.8.5 Material Standards

Wood shingles shall be of naturally durable wood and comply with the requirements of Table 1507.8.5.

TABLE 1507.8.5—WOOD SHINGLE MATERIAL REQUIREMENTS

| MATERIAL | APPLICABLE MINIMUM GRADES | GRADING RULES |
| --- | --- | --- |
| Wood shingles of naturally durable wood | 1, 2 or 3 | CSSB |

CSSB = Cedar Shake and Shingle Bureau

1507.8.6 Attachment

Fasteners for wood shingles shall be corrosion resistant with a minimum penetration of 3/4 inch (19.1 mm) into the sheathing. For sheathing less than 1/2 inch (12.7 mm) in thickness, the fasteners shall extend through the sheathing. Each shingle shall be attached with not fewer than two fasteners.

1507.8.7 Application

Wood shingles shall be laid with a side lap not less than 11/2 inches (38 mm) between joints in adjacent courses, and not be in direct alignment in alternate courses. Spacing between shingles shall be 1/4 to 3/8 inch (6.4 to 9.5 mm). Weather exposure for wood shingles shall not exceed that set in Table 1507.8.7.

TABLE 1507.8.7—WOOD SHINGLE WEATHER EXPOSURE AND ROOF SLOPE

| ROOFING MATERIAL | LENGTH (inches) | GRADE | EXPOSURE (inches) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 3:12 pitch to < 4:12 | 4:12 pitch or steeper |
| Shingles of naturally durable wood | 16 | No. 1 | 3.75 | 5 |
|  |  | No. 2 | 3.5 | 4 |
|  |  | No. 3 | 3 | 3.5 |
|  | 18 | No. 1 | 4.25 | 5.5 |
|  |  | No. 2 | 4 | 4.5 |
|  |  | No. 3 | 3.5 | 4 |
|  | 24 | No. 1 | 5.75 | 7.5 |
|  |  | No. 2 | 5.5 | 6.5 |
|  |  | No. 3 | 5 | 5.5 |

For SI: 1 inch = 25.4 mm.

1507.8.8 Flashing

Diagram

At the juncture of the roof and vertical surfaces, flashing and counterflashing shall be provided in accordance with the manufacturer's installation instructions, and where of metal, shall be not less than 0.019-inch (0.48 mm) (No. 26 galvanized sheet gage) corrosion-resistant metal. The valley flashing shall extend not less than 11 inches (279 mm) from the centerline each way and have a splash diverter rib not less than 1 inch (25 mm) high at the flow line formed as part of the flashing. Sections of flashing shall have an end lap of not less than 4 inches (102 mm). For roof slopes of three units vertical in 12 units horizontal (25-percent slope) and over, the valley flashing shall have a 36-inch-wide (914 mm) underlayment of either one layer of Type I under-layment running the full length of the valley or a self-adhering polymer-modified bitumen sheet bearing a label indicating compliance with ASTM D1970, in addition to other required underlayment. In areas where the average daily temperature in January is 25°F (-4°C) or less or where there is a possibility of ice forming along the eaves causing a backup of water, the metal valley flashing underlayment shall be solidly cemented to the roofing underlayment for slopes under seven units vertical in 12 units horizontal (58-percent slope) or self-adhering polymer-modified bitumen sheet shall be installed.

UpCodes Diagram (1)

Wood Shingles - Valley Flashing

1507.8.9 Label Required

Each bundle of shingles shall be identified by a label of an approved grading or inspection bureau or agency.

1507.9 Wood Shakes

The installation of wood shakes shall comply with the provisions of this section and Table 1507.8.

1507.9.1 Deck Requirements

Wood shakes shall only be used on solid or spaced sheathing. Where spaced sheathing is used, sheathing boards shall be not less than 1-inch by 4-inch (25 mm by 102 mm) nominal dimensions and shall be spaced on centers equal to the weather exposure to coincide with the placement of fasteners. Where 1-inch by 4-inch (25 mm by 102 mm) spaced sheathing is installed at 10 inches (254 mm) on center, additional 1-inch by 4-inch (25 mm by 102 mm) boards shall be installed between the sheathing boards. Where wood shakes are installed over spaced sheathing and the underside of the shakes are exposed to the attic space, the attic shall be ventilated in accordance with Section 1202.2. The shakes shall not be backed with materials that will occupy the required air gap space and prevent the free movement of air on the interior side of the spaced sheathing.

1507.9.1.1 Solid Sheathing Required

Solid sheathing is required in areas where the average daily temperature in January is 25°F (-4°C) or less or where there is a possibility of ice forming along the eaves causing a backup of water.

1507.9.2 Deck Slope

Wood shakes shall only be used on slopes of not less than 4 units vertical in 12 units horizontal (33-percent slope).

1507.9.3 Underlayment

Underlayment shall comply with Section 1507.1.1.

1507.9.4 Ice Barrier

Diagram

Where required, ice barriers shall comply with Section 1507.1.2.

UpCodes Diagram (1)

Ice Barriers: Roofing

1507.9.5 Interlayment

Interlayment shall comply with ASTM D226, Type I.

1507.9.6 Material Standards

Wood shakes shall comply with the requirements of Table 1507.9.6.

TABLE 1507.9.6—WOOD SHAKE MATERIAL REQUIREMENTS

| MATERIAL | MINIMUM GRADES | APPLICABLE GRADING RULES |
| --- | --- | --- |
| Wood shakes of naturally durable wood | 1 | CSSB |
| Taper sawn shakes of naturally durable wood | 1 or 2 | CSSB |
| Preservative-treated shakes and shingles of naturally durable wood | 1 | CSSB |
| Fire-retardant-treated shakes and shingles of naturally durable wood | 1 | CSSB |
| Preservative-treated taper sawn shakes of Southern pine treated in accordance with AWPA U1 (Commodity Specification A, Special Requirement 4.6) | 1 or 2 | TFS |

CSSB = Cedar Shake and Shingle Bureau.

TFS = Forest Products Laboratory of the Texas Forest Services.

1507.9.7 Attachment

Fasteners for wood shakes shall be corrosion resistant with a minimum penetration of 3/4 inch (19.1 mm) into the sheathing. For sheathing less than 1/2 inch (12.7 mm) in thickness, the fasteners shall extend through the sheathing. Each shake shall be attached with not fewer than two fasteners.

1507.9.8 Application

Wood shakes shall be laid with a side lap not less than 11/2 inches (38 mm) between joints in adjacent courses. Spacing between shakes in the same course shall be 3/8 to 5/8 inch (9.5 to 15.9 mm) for shakes and taper sawn shakes of naturally durable wood and shall be 1/4 to 3/8 inch (6.4 to 9.5 mm) for preservative taper sawn shakes. Weather exposure for wood shakes shall not exceed those set in Table 1507.9.8.

TABLE 1507.9.8—WOOD SHAKE WEATHER EXPOSURE AND ROOF SLOPE

| ROOFING MATERIAL | LENGTH (inches) | GRADE | EXPOSURE (inches) 4:12 PITCH OR STEEPER |
| --- | --- | --- | --- |
| Shakes of naturally durable wood | 18 | No. 1 | 7.5 |
|  | 24 | No. 1 | 10a |
| Preservative-treated taper sawn shakes of Southern yellow pine | 18 | No. 1 | 7.5 |
|  | 24 | No. 1 | 10 |
|  | 18 | No. 2 | 5.5 |
|  | 24 | No. 2 | 7.5 |
| Taper sawn shakes of naturally durable wood | 18 | No. 1 | 7.5 |
|  | 24 | No. 1 | 10 |
|  | 18 | No. 2 | 5.5 |
|  | 24 | No. 2 | 7.5 |

For SI: 1 inch = 25.4 mm.

- For 24-inch by 0.375-inch handsplit shakes, the maximum exposure is 7.5 inches.

1507.9.9 Flashing

At the juncture of the roof and vertical surfaces, flashing and counterflashing shall be provided in accordance with the manufacturer's installation instructions, and where of metal, shall be not less than 0.019-inch (0.48 mm) (No. 26 galvanized sheet gage) corrosion-resistant metal. The valley flashing shall extend not less than 11 inches (279 mm) from the centerline each way and have a splash diverter rib not less than 1 inch (25 mm) high at the flow line formed as part of the flashing. Sections of flashing shall have an end lap of not less than 4 inches (102 mm). For roof slopes of three units vertical in 12 units horizontal (25-percent slope) and over, the valley flashing shall have a 36-inch-wide (914 mm) underlayment of either one layer of Type I underlayment running the full length of the valley or a self-adhering polymer-modified bitumen sheet bearing a label indicating compliance with ASTM D1970, in addition to other required underlayment. In areas where the average daily temperature in January is 25°F (-4°C) or less or where there is a possibility of ice forming along the eaves causing a backup of water, the metal valley flashing underlayment shall be solidly cemented to the roofing underlayment for slopes under seven units vertical in 12 units horizontal (58-percent slope) or self-adhering polymer-modified bitumen sheet shall be installed.

1507.9.10 Label Required

Each bundle of shakes shall be identified by a label of an approved grading or inspection bureau or agency.

1507.10 Built-Up Roofs

Diagram

The installation of built-up roofs shall comply with the provisions of this section.

UpCodes Diagram (1)

Primary

Built-Up Roof at Roof-to-Wall Expansion Joint

1507.10.1 Slope

Built-up roofs shall have a design slope of not less than 1/4 unit vertical in 12 units horizontal (2-percent slope) for drainage, except for coal-tar built-up roofs that shall have a design slope of not less than 1/8 unit vertical in 12 units horizontal (1-percent slope).

1507.10.2 Material Standards

Built-up roof covering materials shall comply with the standards in Table 1507.10.2 or UL 55A.

TABLE 1507.10.2—BUILT-UP ROOFING MATERIAL STANDARDS

| MATERIAL STANDARD | STANDARD |
| --- | --- |
| Acrylic coatings used in roofing | ASTM D6083 |
| Aggregate surfacing | ASTM D1863 |
| Asphalt adhesive used in roofing | ASTM D3747 |
| Asphalt cements used in roofing | ASTM D2822; D3019; D4586 |
| Asphalt-coated glass fiber base sheet | ASTM D4601 |
| Asphalt coatings used in roofing | ASTM D1227; D2823; D2824; D4479 |
| Asphalt glass felt | ASTM D2178 |
| Asphalt primer used in roofing | ASTM D41 |
| Asphalt-saturated and asphalt-coated organic felt base sheet | ASTM D2626 |
| Asphalt-saturated organic felt (perforated) | ASTM D226 |
| Asphalt used in roofing | ASTM D312 |
| Coal-tar cements used in roofing | ASTM D4022; D5643 |
| Coal-tar saturated organic felt | ASTM D227 |
| Coal-tar pitch used in roofing | ASTM D450; Type I or II |
| Coal-tar primer used in roofing, dampproofing and waterproofing | ASTM D43 |
| Glass mat, coal-tar | ASTM D4990 |
| Glass mat, venting type | ASTM D4897 |
| Mineral-surfaced inorganic cap sheet | ASTM D3909 |
| Thermoplastic fabrics used in roofing | ASTM D5665, D5726 |

1507.11 Modified Bitumen Roofing

Diagram

The installation of modified bitumen roofing shall comply with the provisions of this section.

UpCodes Diagram (1)

Primary

Example: Built-Up Roofing with Modified Cap Sheet

1507.11.1 Slope

Modified bitumen roofing shall have a design slope of not less than 1/4 unit vertical in 12 units horizontal (2-percent slope) for drainage.

1507.11.2 Material Standards

Modified bitumen roofing materials shall comply with ASTM D6162, ASTM D6163, ASTM D6164, ASTM D6222, ASTM D6223, ASTM D6298 or ASTM D6509.

1507.11.2.1 Base Sheet

A base sheet that complies with the requirements of Section 1507.11.2, ASTM D1970 or ASTM D4601 shall be permitted to be used with a modified bitumen cap sheet.

1507.12 Single-Ply Roofing

The installation of single-ply roofing shall comply with the provisions of this section.

1507.12.1 Slope

Single-ply membrane roofs shall have a design slope of not less than 1/4 unit vertical in 12 units horizontal (2-percent slope) for drainage.

1507.12.2 Material Standards

Single-ply roof coverings shall comply with the material standards in Table 1507.12.2.

TABLE 1507.12.2—SINGLE-PLY ROOFING MATERIAL STANDARDS

| MATERIAL | MATERIAL STANDARD |
| --- | --- |
| Chlorosulfonated polyethylene (CSPE) or polyisobutylene (PIB) | ASTM D5019 |
| Ethylene propylene diene monomer (EPDM) | ASTM D4637 |
| Ketone Ethylene Ester (KEE) | ASTM D6754 |
| Polyvinyl Chloride (PVC) or (PVC/KEE) | ASTM D4434 |
| Thermoplastic polyolefin (TPO) | ASTM D6878 |

1507.12.3 Ballasted Low-Slope Roofs

Ballasted low-slope roofs shall be installed in accordance with this section and Section 1504.5. Stone used as ballast shall comply with ASTM D448 or ASTM D7655.

1507.13 Sprayed Polyurethane Foam Roofing

The installation of sprayed polyurethane foam roofing shall comply with the provisions of this section.

1507.13.1 Slope

Sprayed polyurethane foam roofs shall have a design slope of not less than 1/4 unit vertical in 12 units horizontal (2-percent slope) for drainage.

1507.13.2 Material Standards

Spray-applied polyurethane foam insulation shall comply with ASTM C1029 Type III or IV or ASTM D7425.

1507.13.3 Application

Foamed-in-place roof insulation shall be installed in accordance with the manufacturer's instructions. A liquid-applied protective coating that complies with Table 1507.13.3 shall be applied not less than 2 hours nor more than 72 hours following the application of the foam.

TABLE 1507.13.3—PROTECTIVE COATING MATERIAL STANDARDS

| MATERIAL | STANDARD |
| --- | --- |
| Acrylic coating | ASTM D6083 |
| Silicone coating | ASTM D6694 |
| Moisture-cured polyurethane coating | ASTM D6947 |

1507.13.4 Foam Plastics

Foam plastic materials and installation shall comply with Chapter 26.

1507.14 Liquid-Applied Roofing

Diagram

The installation of liquid-applied roofing shall comply with the provisions of this section.

UpCodes Diagrams (3)

Primary

Example: Liquid-Applied Roofing at Rooftop Unit Curb Support

Primary

Example: Liquid-Applied Roofing at Pipe / Conduit Penetrations

Primary

Example: Liquid-Applied Roofing at Duct Penetration

1507.14.1 Slope

Liquid-applied roofing shall have a design slope of not less than 1/4 unit vertical in 12 units horizontal (2-percent slope).

1507.14.2 Material Standards

Liquid-applied roofing shall comply with ASTM C836, ASTM C957 or ASTM D3468.

1507.14.3 Application

Liquid-applied roofing shall be installed in accordance with this chapter and the manufacturer's installation instructions.

1507.15 Vegetative Roofs and Landscaped Roofs

Vegetative roofs and landscaped roofs shall comply with the requirements of this chapter, Section 1607.14 and the California Fire Code.

Building products (4)

FOAMULAR® & FOAMULAR® NGX® Thermapink® 25 Tapered

Owens Corning

FOAMULAR® NGX® 400/600/1000

Owens Corning

FOAMULAR® NGX® 404RB/604RB

Owens Corning

FOAMULAR® NGX® THERMAPINK® 25 XPS

Owens Corning

[BF] 1507.15.1 Structural Fire Resistance

The structural frame and roof construction supporting the load imposed on the roof by the vegetative roof or landscaped roofs shall comply with the fire-resistance-rating requirements of Table 601.

1507.16 BIPV Shingles

The installation of building-integrated photovoltaic (BIPV) shingles shall comply with the provisions of this section.

1507.16.1 Deck Requirements

BIPV shingles shall be applied to a solid or closely fitted deck, except where the shingles are specifically designed to be applied over spaced sheathing.

1507.16.2 Deck Slope

BIPV shingles shall be installed on roof slopes of not less than 2 units vertical in 12 units horizontal (2:12).

1507.16.3 Underlayment

Underlayment shall comply with Section 1507.1.1.

1507.16.4 Ice Barrier

Diagram

Where required, ice barriers shall comply with Section 1507.1.2.

UpCodes Diagram (1)

Ice Barriers: Roofing

1507.16.5 Fasteners

Fasteners for BIPV shingles shall be galvanized, stainless steel, aluminum or copper roofing nails, minimum 12-gage [0.105 inch (2.67 mm)] shank with a minimum 3/8-inch-diameter (9.5 mm) head, of a length to penetrate through the roofing materials and not less than 3/4 inch (19.1 mm) into the roof sheathing. Where the roof sheathing is less than 3/4 inch (19.1 mm) thick, the nails shall penetrate through the sheathing. Fasteners shall comply with ASTM F1667.

1507.16.6 Material Standards

BIPV shingles shall be listed and labeled in accordance with UL 7103.

1507.16.7 Attachment

BIPV shingles shall be attached in accordance with the manufacturer's installation instructions.

1507.16.8 Wind Resistance

BIPV shingles shall comply with the classification requirements of Table 1504.2 for the appropriate maximum basic wind speed.

1507.16.9 Flashing

Flashing for BIPV shingles shall be installed in accordance with the roof covering manufacturer's installation instructions to prevent water from entering the wall and roof through joints in copings, through moisture-permeable materials and at intersections with parapet walls and other penetrations through the roof plane.

1507.17 Building-Integrated Photovoltaic Roof Panels

The installation of BIPV roof panels shall comply with the provisions of this section.

1507.17.1 Deck Requirements

BIPV roof panels shall be applied to a solid or closely fitted deck, except where the roof covering is specifically designed to be applied over spaced sheathing.

1507.17.2 Deck Slope

BIPV roof panels shall be used only on roof slopes of 2 units vertical in 12 units horizontal (2:12) or greater.

1507.17.3 Underlayment

Underlayment shall comply with ASTM D226, ASTM D4869 or ASTM D6757.

1507.17.4 Underlayment Application

Underlayment shall be applied shingle fashion, parallel to and starting from the eave, lapped 2 inches (51 mm) and fastened sufficiently to hold in place.

1507.17.4.1 High-Wind Attachment

Underlayment applied in areas subject to high winds [Vasd greater than 110 mph (49 m/s) as determined in accordance with Section 1609.3.1] shall be applied in accordance with the manufacturer's instructions. Fasteners shall be applied along the overlap at not more than 36 inches (914 mm) on center. Underlayment installed where Vasd is not less than 120 mph (54 m/s) shall comply with ASTM D226, Type III, ASTM D4869, Type IV or ASTM D6757. The underlayment shall be attached in a grid pattern of 12 inches (305 mm) between side laps with a 6-inch (152 mm) spacing at the side laps. The underlayment shall be applied in accordance with Section 1507.1.1 except all laps shall be not less than 4 inches (102 mm). Underlayment shall be attached using cap nails or cap staples. Caps shall be metal or plastic with a nominal head diameter of not less than 1 inch (25.4 mm). Metal caps shall have a thickness of not less than 0.010 inch (0.25 mm). Power-driven metal caps shall have a thickness of not less than 0.010 inch (0.25 mm). Thickness of the outside edge of plastic caps shall be not less than 0.035 inch (0.89 mm). The cap nail shank shall be not less than 0.083 inch (2.11 mm) for ring shank cap nails and 0.091 inch (2.31 mm) for smooth shank cap nails. Staple gage shall be not less than 21 gage [0.0.2 inch (0.81 mm)]. Cap nail shank and cap staple legs shall have a length sufficient to penetrate through-the-roof sheathing or not less than 3/4 inch (19.1 mm) into the roof sheathing.

Exception: As an alternative, adhered underlayment complying with ASTM D1970 shall be permitted.

1507.17.4.2 Ice Barrier

Diagram

In areas where there has been a history of ice forming along the eaves causing a backup of water, an ice barrier consisting of not fewer than two layers of underlayment cemented together or of a self-adhering polymer modified bitumen sheet shall be used instead of normal underlayment and extend from the lowest edges of all roof surfaces to a point not less than 24 inches (610 mm) inside the exterior wall line of the building.

Exception: Detached accessory structures that do not contain conditioned floor area.

UpCodes Diagram (1)

Ice Barriers: Roofing

1507.17.5 Material Standards

BIPV roof panels shall be listed and labeled in accordance with UL 7103.

1507.17.6 Attachment

BIPV roof panels shall be attached in accordance with the manufacturer's installation instructions.

1507.17.7 Flashing

Flashing for BIPV roof panels shall be installed in accordance with the roof covering manufacturer's installation instructions to prevent water from entering the wall and roof through joints in copings, through moisture-permeable materials and at intersections with parapet walls and other penetrations through the roof plane.

Section 1508 Roof Insulation

[BF] 1508.1 General

Diagram

The use of above-deck thermal insulation shall be permitted provided that such insulation is covered with an approved roof covering and passes the tests of NFPA 276 or UL 1256 when tested as an assembly.

Exceptions:

- Foam plastic roof insulation shall conform to the material and installation requirements of Chapter 26.

- Where a concrete or composite metal and concrete roof deck is used and the above-deck thermal insulation is covered with an approved roof covering.

UpCodes Diagrams (3)

Example: Liquid-Applied Roofing at Duct Penetration

Example: Liquid-Applied Roofing at Pipe / Conduit Penetrations

Example: Liquid-Applied Roofing at Rooftop Unit Curb Support

Building products (6)

FOAMGLAS® T3+ Cellular Glass

Owens Corning

FOAMULAR® & FOAMULAR® NGX® Thermapink® 25 Tapered

Owens Corning

FOAMULAR® NGX® 400/600/1000

Owens Corning

FOAMULAR® NGX® 404RB/604RB

Owens Corning

FOAMULAR® NGX® THERMAPINK® 25 XPS

Owens Corning

GreenGirt CMH Roof™

Advanced Architectural Products

[BF] 1508.2 Material Standards

Above-deck thermal insulation board shall comply with the standards in Table 1508.2.

[BF] TABLE 1508.2—MATERIAL STANDARDS FOR ROOF INSULATION

| MATERIAL | STANDARD |
| --- | --- |
| Cellular glass board | ASTM C552 or ASTM C1902 |
| Composite boards | ASTM C1289, Type III, IV, V or VII |
| Expanded polystyrene | ASTM C578 |
| Extruded polystyrene | ASTM C578 |
| Fiber-reinforced gypsum board | ASTM C1278 |
| Glass-faced gypsum board | ASTM C1177 |
| High-density polyisocyanurate board | ASTM C1289, Type II, Class 4 |
| Mineral fiber insulation board | ASTM C726 |
| Perlite board | ASTM C728 |
| Polyisocyanurate board | ASTM C1289, Type I or II |
| Wood fiberboard | ASTM C208, Type II |

Section 1509 Roof Coatings

1509.1 General

The installation of a roof coating on a roof covering shall comply with the requirements of Section 1505 and this section.

1509.2 Material Standards

Roof coating materials shall comply with the standards in Table 1509.2.

TABLE 1509.2—ROOF COATING MATERIAL STANDARDS

| MATERIAL | STANDARD |
| --- | --- |
| Acrylic coating | ASTM D6083 |
| Asphaltic emulsion coating | ASTM D1227 |
| Asphalt coating | ASTM D2823 |
| Asphalt roof coating | ASTM D4479 |
| Aluminum-pigmented asphalt coating | ASTM D2824 |
| Silicone coating | ASTM D6694 |
| Moisture-cured polyurethane coating | ASTM D6947 |

Section 1510 Radiant Barriers Installed Above Deck

[BF] 1510.1 General

A radiant barrier installed above a deck shall comply with Sections 1510.2 through 1510.4.

[BF] 1510.2 Fire Testing

Radiant barriers shall be permitted for use above decks where the radiant barrier is covered with an approved roof covering and the system consisting of the radiant barrier and the roof covering complies with the requirements of either FM 4450 or UL 1256.

[BF] 1510.3 Installation

The low emittance surface of the radiant barrier shall face the continuous airspace between the radiant barrier and the roof covering.

[BF] 1510.4 Material Standards

A radiant barrier installed above a deck shall comply with ASTM C1313/C1313M.

Section 1511 Rooftop Structures

Diagram

UpCodes Diagrams (7)

Rooftop Structures and Penthouses

Obstructions on Roofs

Occupied Roofs

Shaft Enclosure at the Top

Roof Assemblies at Interior Exit Stairs (2024)

Rooftop Structures: Fire-Resistance Ratings

Mechanical Equipment Screens

[BG] 1511.1 General

The provisions of this section shall govern the construction of rooftop structures.

1511.1.1 Area Limitation

The aggregate area of penthouses and other enclosed rooftop structures shall not exceed one-third the area of the supporting roof deck. Such penthouses and other enclosed rooftop structures shall not be required to be included in determining the building height, number of stories or building area as regulated by Section 503.1. The area of such penthouses shall not be included in determining the fire area specified in Section 901.7.

[BG] 1511.2 Penthouses

Diagram

Penthouses in compliance with Sections 1511.2.1 through 1511.2.4 shall be considered as a portion of the story directly below the roof deck on which such penthouses are located. Other penthouses shall be considered as an additional story of the building.

UpCodes Diagrams (3)

Primary

Rooftop Structures and Penthouses

Egress From Stories or Occupied Roof

Penthouse Stair Access to Roof

[BG] 1511.2.1 Height Above Roof Deck

Diagram

Penthouses constructed on buildings of other than Type I construction shall not exceed 18 feet (5486 mm) in height above the roof deck as measured to the average height of the roof of the penthouse. Penthouses located on the roof of buildings of Type I construction shall not be limited in height.

Exception: Where used to enclose tanks or elevators that travel to the roof level, penthouses shall be permitted to have a maximum height of 28 feet (8534 mm) above the roof deck.

UpCodes Diagram (1)

Primary

Penthouse Stair Access to Roof

[BG] 1511.2.2 Use Limitations

Penthouses shall not be used for purposes other than the shelter of mechanical or electrical equipment, tanks, elevators and related machinery, stairways or vertical shaft openings in the roof assembly, including ancillary spaces used to access elevators and stairways.

[BG] 1511.2.3 Weather Protection

Provisions such as louvers, louver blades or flashing shall be made to protect the mechanical and electrical equipment and the building interior from the elements.

[BG] 1511.2.4 Type of Construction

Diagram

Penthouses shall be constructed of building element materials as required for the type of construction of the building. Penthouse exterior walls and roof construction shall have a fire-resistance rating as required for the type of construction of the building. Supporting construction of such exterior walls and roof construction shall have a fire-resistance rating not less than required for the exterior wall or roof supported.

Exceptions:

- On buildings of Type I construction, the exterior walls and roofs of penthouses with a fire separation distance greater than 5 feet (1524 mm) and less than 20 feet (6096 mm) shall be permitted to have not less than a 1-hour fire-resistance rating. The exterior walls and roofs of penthouses with a fire separation distance of 20 feet (6096 mm) or greater shall not be required to have a fire-resistance rating.

- On buildings of Type I construction two stories or less in height above grade plane or of Type II construction, the exterior walls and roofs of penthouses with a fire separation distance greater than 5 feet (1524 mm) and less than 20 feet (6096 mm) shall be permitted to have not less than a 1-hour fire-resistance rating or a lesser fire-resistance rating as required by Table 705.5 and be constructed of fire-retardant-treated wood. The exterior walls and roofs of penthouses with a fire separation distance of 20 feet (6096 mm) or greater shall be permitted to be constructed of fire-retardant-treated wood and shall not be required to have a fire-resistance rating. Interior framing and walls shall be permitted to be constructed of fire-retardant-treated wood.

- On buildings of Type III, IV or V construction, the exterior walls of penthouses with a fire separation distance greater than 5 feet (1524 mm) and less than 20 feet (6096 mm) shall be permitted to have not less than a 1-hour fire-resistance rating or a lesser fire-resistance rating as required by Table 705.5. On buildings of Type III, IV or VA construction, the exterior walls of penthouses with a fire separation distance of 20 feet (6096 mm) or greater shall be permitted to be of heavy timber construction complying with Sections 602.4 and 2304.11 or noncombustible construction or fire-retardant-treated wood and shall not be required to have a fire-resistance rating.

UpCodes Diagram (1)

Rooftop Structures and Penthouses

[BG] 1511.3 Tanks

Diagram

Tanks having a capacity of more than 500 gallons (1893 L) located on the roof deck of a building shall be supported on masonry, reinforced concrete, steel or heavy timber construction complying with Section 2304.11 provided that, where such supports are located in the building above the lowest story, the support shall be fire-resistance rated as required for Type IA construction.

UpCodes Diagram (1)

Example: Liquid-Applied Roofing at Rooftop Unit Curb Support

[BG] 1511.3.1 Valve and Drain

In the bottom or on the side near the bottom of the tank, a pipe or outlet, fitted with a suitable quick-opening valve for discharging the contents into a drain in an emergency shall be provided.

[BG] 1511.3.2 Location

Tanks shall not be placed over or near a stairway or an elevator shaft, unless there is a solid roof or floor underneath the tank.

[BG] 1511.3.3 Tank Cover

Unenclosed roof tanks shall have covers sloping toward the perimeter of the tanks.

[BG] 1511.4 Cooling Towers

Cooling towers located on the roof deck of a building and greater than 250 square feet (23.2 m2) in base area or greater than 15 feet (4572 mm) in height above the roof deck, as measured to the highest point on the cooling tower, where the roof is greater than 50 feet (15 240 mm) in height above grade plane shall be constructed of noncombustible materials. The base area of cooling towers shall not exceed one-third the area of the supporting roof deck.

Exception: Drip boards and the enclosing construction shall be permitted to be of wood not less than 1 inch (25 mm) nominal thickness, provided that the wood is covered on the exterior of the tower with noncombustible material.

Building product (1)

FOAMGLAS® T3+ Cellular Glass

Owens Corning

[BG] 1511.5 Towers, Spires, Domes and Cupolas

Towers, spires, domes and cupolas shall be of a type of construction having fire-resistance ratings not less than required for the building on top of which such tower, spire, dome or cupola is built. Towers, spires, domes and cupolas greater than 85 feet (25 908 mm) in height above grade plane as measured to the highest point on such structures, and either greater than 200 square feet (18.6 m2) in horizontal area or used for any purpose other than a belfry or an architectural embellishment, shall be constructed of and supported on Type I or II construction.

[BG] 1511.5.1 Noncombustible Construction Required

Towers, spires, domes and cupolas greater than 60 feet (18 288 mm) in height above the highest point at which such structure contacts the roof as measured to the highest point on such structure, or that exceeds 200 square feet (18.6 m2) in area at any horizontal section, or which is intended to be used for any purpose other than a belfry or architectural embellishment, or is located on the top of a building greater than 50 feet (1524 mm) in building height shall be constructed of and supported by noncombustible materials and shall be separated from the building below by construction having a fire-resistance rating of not less than 1.5 hours with openings protected in accordance with Section 711. Such structures located on the top of a building greater than 50 feet (15 240 mm) in building height shall be supported by noncombustible construction.

[BG] 1511.5.2 Towers and Spires

Enclosed towers and spires shall have exterior walls constructed as required for the building on top of which such towers and spires are built. The roof covering of spires shall be not less than the same class of roof covering required for the building on top of which the spire is located.

[BG] 1511.6 Mechanical Equipment Screens

Diagram

Mechanical equipment screens shall be constructed of the materials specified for the exterior walls in accordance with the type of construction of the building. Where the fire separation distance is greater than 5 feet (1524 mm), mechanical equipment screens shall not be required to comply with the fire-resistance rating requirements.

UpCodes Diagram (1)

Primary

Mechanical Equipment Screens

[BG] 1511.6.1 Height Limitations

Mechanical equipment screens shall not exceed 18 feet (5486 mm) in height above the roof deck, as measured to the highest point on the mechanical equipment screen.

Exception: Where located on buildings of Type IA construction, the height of mechanical equipment screens shall not be limited.

[BG] 1511.6.2 Type I, II, III or IV Construction

Regardless of the requirements in Section 1511.6, mechanical equipment screens that are located on the roof decks of buildings of Type I, II, III or IV construction shall be permitted to be constructed of combustible materials in accordance with any one of the following limitations:

- The fire separation distance shall be not less than 20 feet (6096 mm) and the height of the mechanical equipment screen above the roof deck shall not exceed 4 feet (1219 mm) as measured to the highest point on the mechanical equipment screen.

- The fire separation distance shall be not less than 20 feet (6096 mm) and the mechanical equipment screen shall be constructed of fire-retardant-treated wood complying with Section 2303.2 for exterior installation.

- Where exterior wall covering panels are used, the panels shall have a flame spread index of 25 or less when tested in the minimum and maximum thicknesses intended for use, with each face tested independently in accordance with ASTM E84 or UL 723. The panels shall be tested in the minimum and maximum thicknesses intended for use in accordance with, and shall comply with the acceptance criteria of, NFPA 285 and shall be installed as tested. Where the panels are tested as part of an exterior wall assembly in accordance with NFPA 285, the panels shall be installed on the face of the mechanical equipment screen supporting structure in the same manner as they were installed on the tested exterior wall assembly.

[BG] 1511.6.3 Type V Construction

The height of mechanical equipment screens located on the roof decks of buildings of Type V construction, as measured from grade plane to the highest point on the mechanical equipment screen, shall be permitted to exceed the maximum building height allowed for the building by other provisions of this code where complying with any one of the following limitations, provided that the fire separation distance is greater than 5 feet (1524 mm):

- Where the fire separation distance is not less than 20 feet (6096 mm), the height above grade plane of the mechanical equipment screen shall not exceed 4 feet (1219 mm) more than the maximum building height allowed.

- The mechanical equipment screen shall be constructed of noncombustible materials.

- The mechanical equipment screen shall be constructed of fire-retardant-treated wood complying with Section 2303.2 for exterior installation.

- Where the fire separation distance is not less than 20 feet (6096 mm), the mechanical equipment screen shall be constructed of materials having a flame spread index of 25 or less when tested in the minimum and maximum thicknesses intended for use with each face tested independently in accordance with ASTM E84 or UL 723.

[BG] 1511.7 Other Rooftop Structures

Rooftop structures not regulated by Sections 1511.2 through 1511.6 shall comply with Sections 1511.7.1 through 1511.7.6, as applicable.

[BG] 1511.7.1 Aerial Supports

Aerial supports shall be constructed of noncombustible materials.

Exception: Aerial supports not greater than 12 feet (3658 mm) in height as measured from the roof deck to the highest point on the aerial supports shall be permitted to be constructed of combustible materials.

[BG] 1511.7.2 Bulkheads

Bulkheads used for the shelter of mechanical or electrical equipment or vertical shaft openings in the roof assembly shall comply with Section 1511.2 as penthouses. Bulkheads used for any other purpose shall be considered as an additional story of the building.

[BG] 1511.7.3 Dormers

Diagram

Dormers shall be of the same type of construction as required for the roof in which such dormers are located or the exterior walls of the building.

UpCodes Diagrams (2)

Example: Roof Openings - Shed Dormer

Example: Roof Openings - Gable Dormer

[BG] 1511.7.4 Fences

Fences and similar structures shall comply with Section 1511.6 as mechanical equipment screens.

[BG] 1511.7.5 Flagpoles

Flagpoles and similar structures shall not be required to be constructed of noncombustible materials and shall not be limited in height or number.

[BG] 1511.7.6 Lightning Protection Systems

Lightning protection system components shall be installed in accordance with Sections 1511.7.6.1, 1511.7.6.2 and 2703.

[BG] 1511.7.6.1 Installation on Metal Edge Systems or Gutters

Lightning protection system components attached to ANSI/SPRI/FM 4435/ES-1 or ANSI/SPRI GT-1 tested metal edge systems or gutters shall be installed with compatible brackets, fasteners or adhesives, in accordance with the metal edge systems or gutter manufacturer's installation instructions. Where the metal edge system or gutter manufacturer is unknown, installation shall be as directed by a registered design professional.

[BG] 1511.7.6.2 Installation on Roof Coverings

Lightning protection system components directly attached to or through the roof covering shall be installed in accordance with this chapter and the roof covering manufacturer's installation instructions. Flashing shall be installed in accordance with the roof assembly manufacturer's installation instructions and Sections 1503.2 and 1507 where the lightning protection system installation results in a penetration through the roof covering. Where the roof covering manufacturer is unknown, installation shall be as directed by a registered design professional.

[BG] 1511.8 Structural Fire Resistance

Diagram

The structural frame and roof construction supporting loads imposed upon the roof by any rooftop structure shall comply with the requirements of Table 601. The fire-resistance reduction permitted by Table 601, Note a, shall not apply to roofs containing rooftop structures.

UpCodes Diagram (1)

Primary

Rooftop Structures: Fire-Resistance Ratings

[BG] 1511.9 Raised-Deck Systems Installed Over a Roof Assembly

Raised-deck systems installed above a roof assembly shall comply with Sections 1511.9.1 through 1511.9.5.

[BG] 1511.9.1 Installation

The installation of a raised-deck system shall comply with all of the following:

- The perimeter of the raised-deck system shall be surrounded on all sides by walls or by a noncombustible enclosure approved to prevent fire intrusion below the raised-deck system. The wall or enclosure shall extend at least from the roof assembly to the top surface of the raised-deck system. The enclosure shall not impede roof drainage in accordance with Section 1511.9.5.

- A raised-deck system shall be installed above a listed roof assembly.

Exception: Where the roof assembly is not required to have a fire classification in accordance with Section 1505.2.

- A raised-deck system shall be installed in accordance with the manufacturer's installation instructions.

- A raised-deck system shall not impede the operation of plumbing or mechanical vents, exhaust, air inlets or roof drains. Where required, access for inspection, cleaning or maintenance shall be provided.

[BG] 1511.9.2 Fire Classification

The raised-deck system shall be listed and identified with a fire classification in accordance with Section 1505 and shall be tested in accordance with either Section 1511.9.2.1 or 1511.9.2.2.

[BG] 1511.9.2.1 Fire Testing of the Raised Deck System Installed Over a Classified Roof Assembly

The raised-deck system shall be tested separately from the roof assembly over which it is installed. The fire classification of the raised-deck system shall be not less than the fire classification for the roof assembly over which it is installed.

Exception: Where the decking or pavers of the raised-deck system consists of brick, masonry, concrete or other noncombustible materials, fire testing of the raised-deck system is not required.

[BG] 1511.9.2.2 Fire Testing of the Raised Deck System Together With the Roof Assembly

The roof assembly and the raised-deck system shall be tested together.

[BG] 1511.9.3 Pedestals or Supports

The pedestals or supports for the raised-deck system shall be installed in accordance with manufacturer's installation instructions.

[BG] 1511.9.4 Structural Requirements

The raised-deck system shall be designed for all applicable loads in accordance with Chapter 16 and performance requirements in Section 1504.5.

[BG] 1511.9.5 Roof Drainage

The raised-deck system, including the wall or enclosure between the roof assembly and the raised deck, shall be designed and installed to allow for the operation of the roof drainage system as required by Section 1502 and the California Plumbing Code. The roof structure shall be designed to support any standing water resulting from the installation of the raised-deck system.

[BG] 1511.9.6 Accessibility and Egress

The raised-deck system shall be accessible in accordance with Chapters 11A or 11B, as applicable, and means of egress shall be provided in accordance with Chapter 10.

1511.10 Photovoltaic (PV) Panel Systems

[BSC, DSA-SS, DSA-SS/CC, HCD 1 & HCD 2, OSHPD 1, 1R, 2, 4 & 5] Rooftop-mounted photovoltaic panels and modules shall be listed and labeled in accordance with Section 3111.3.1 and installed in accordance with the manufacturer's printed installation instructions.

1511.10.1 Design

[BSC, DSA-SS, DSA-SS/CC, HCD 1 & HCD 2, OSHPD 1, 1R, 2, 4 & 5] Supports and attachments of photovoltaic panels to the roof structure, the panels, modules and components shall be designed in accordance with this code.

- Seismic design shall be in accordance with ASCE 7 Section 13.6.12.

- Wind design pressures shall be determined from ASCE 7 Section 29.4.3 or 29.4.4 using effective wind area in accordance with ASCE 7 Section 26.2.

Construction documents for the supports and attachments shall be submitted to the enforcement agency for approval.

Section 1512 Reroofing

1512.1 General

Materials and methods of application used for recovering or replacing an existing roof covering shall comply with the requirements of Chapter 15.

Exceptions:

- Roof replacement or roof recover of existing low-slope roof coverings shall not be required to meet the minimum design slope requirement of 1/4 unit vertical in 12 units horizontal (2-percent slope) in Section 1507 for roofs that provide positive roof drainage and meet the requirements of Sections 1608.3 and 1611.2.

- Recovering or replacing an existing roof covering shall not be required to meet the requirement for secondary (emergency overflow) drains or scuppers in Section 1502.2 for roofs that provide for positive roof drainage and meet the requirements of Sections 1608.3 and 1611.2. For the purposes of this exception, existing secondary drainage or scupper systems required in accordance with this code shall not be removed unless they are replaced by secondary drains or scuppers designed and installed in accordance with Section 1502.2.

1512.2 Roof Replacement

Roof replacement shall include the removal of all existing layers of roof assembly materials down to the roof deck.

Exceptions:

- Where the existing roof assembly includes an ice barrier membrane that is adhered to the roof deck and the existing sheathing is not water-soaked or deteriorated to the point that it is not adequate as a base for additional roofing, the existing ice barrier membrane shall be permitted to remain in place and covered with an additional layer of ice barrier membrane in accordance with Section 1507 where permitted by the roof covering manufacturer and new ice barrier underlayment manufacturer.

- Where the existing roof includes a self-adhered underlayment and the existing sheathing is not water-soaked or deteriorated to the point that it is not adequate as a base for additional roofing, the existing self-adhered underlayment shall be permitted to remain in place and covered with an underlayment complying with Tables 1507.1.1(1), 1507.1.1(2) and 1507.1.1(3).

- Where the existing roof includes one layer of self-adhered underlayment and the existing layer cannot be removed without damaging the roof deck, a second layer of self-adhered underlayment is permitted to be installed over the existing self-adhered underlayment provided that the following conditions are met:

  - It is permitted by the roof covering manufacturer and self-adhered underlayment manufacturer.

  - The existing sheathing is not water-soaked or deteriorated to the point that it is not adequate as a base for additional roofing.

  - The second layer of self-adhered underlayment is installed such that buildup of material at walls, valleys, roof edges, end laps and side laps does not exceed two layers.

1512.3 Roof Recover

The installation of a new roof covering over an existing roof covering shall be permitted where any of the following conditions occur:

- Where the new roof covering is installed in accordance with the roof covering manufacturer's approved instructions.

- Complete and separate roofing systems, such as standing-seam metal roof panel systems, that are designed to transmit the roof loads directly to the building's structural system and that do not rely on existing roofs and roof coverings for support, shall not require the removal of existing roof coverings.

- Metal panel, metal shingle and concrete and clay tile roof coverings shall be permitted to be installed over existing wood shake roofs when applied in accordance with Section 1512.3.1.

- The application of a new protective roof coating over an existing protective roof coating, metal roof panel, built-up roof, spray polyurethane foam roofing system, metal roof shingles, mineral-surfaced roll roofing, modified bitumen roofing or thermoset and thermoplastic single-ply roofing shall be permitted without tear off of existing roof coverings.

Exception: A roof recover shall not be permitted where any of the following conditions occur:

- The existing roof or roof covering is water-soaked or has deteriorated to the point that the existing roof or roof covering is not adequate as a base for additional roofing.

- The existing roof covering is slate, clay, cement or asbestos-cement tile.

- The existing roof has two or more applications of any type of roof covering.

1512.3.1 Roof Recovering Over Wood Shingles or Shakes

Where the application of a new roof covering over wood shingle or shake roofs creates a combustible concealed space, the entire existing surface shall be covered with gypsum panel products, mineral fiber, glass fiber or other approved materials securely fastened in place.

1512.4 Reinstallation of Materials

Existing slate, clay or cement tile shall be permitted for reinstallation, except that damaged, cracked or broken slate or tile shall not be reinstalled. Existing vent flashing, metal edgings, drain outlets, collars and metal counterflashings shall not be reinstalled where rusted, damaged or deteriorated. Existing ballast that is damaged, cracked or broken shall not be reinstalled. Existing aggregate surfacing materials from built-up roofs shall not be reinstalled.

1512.5 Flashings

Flashings shall be reconstructed in accordance with approved manufacturer's installation instructions. Metal flashing to which bituminous materials are to be adhered shall be primed prior to installation.

Section 1513 [DSA-SS & DSA-SS/CC, OSHPD 1, 1R, 2, 4 & 5] Seismic Anchorage of Slate Shingle, Clay and Concrete Tile Roof Coverings

1513.1 Fasteners

Nails shall be long enough to penetrate into the sheathing 3/4 inch (19 mm). Where sheathing is less than 3/4 inch (19 mm) in thickness, nails shall be driven into supports, unless nails with ring shanks are used.

All fasteners shall be corrosion resistant and fabricated of copper, stainless steel or brass, or shall have a hot dipped galvanized coating not less than 1.0 ounce of zinc per square foot (305 gm/m2).

Nails for slate shingles and clay or concrete tile shall be copper, brass or stainless steel with gage and length per common ferrous nails.

1513.2 Wire

Wire for attaching slate shingles and clay or concrete tile shall be copper, brass or stainless steel capable of supporting four times the weight of tile.

Wire supporting a single tile or shingle shall not be smaller than 1/16 inch (1.6 mm) in diameter. Continuous wire ties supporting more than one tile shall not be smaller than 0.084 inch (2 mm) in diameter.

1513.3 Metal Strips

Metal strips for attaching slate shingles and clay or concrete tile shall be copper, brass or stainless steel capable of supporting four times the weight of tile.

1513.4 Clay or Concrete Tiles

Clay or concrete tile shall be installed in accordance with Table 1507.3.7 and as described herein.

- On wood roofs or roofs of other material to which wood strips are secured, every cover or top tile when fastened with nails shall be nailed directly into 11/4 inches (32 mm) sound grain soft wood strips of sufficient height to support the tile.

Pan or bottom tiles shall be nailed directly to the roof sheathing or to wood strips. Wood strips shall be secured to the roof by nails spaced not over 12 inches (305 mm) apart.

- On concrete roofs, wires shall be secured in place by wire loops embedded into the concrete not less than 2 inches (51 mm). The wire loops shall be spaced not more than 36 inches (914 mm) on center parallel to the eaves, and spaced vertically to allow for the minimum 3 inches (76 mm) lapping of the tile.

- Where continuous ties of twisted wire, interlocking wires or metal strips extending from the ridge to eave are used to attach tile, the ties shall be attached to the roof construction at the ridge, eave and at intervals not exceeding 10 feet 0 inch (3048 mm) on center. The ties within 2 feet 0 inch (610 mm) of the rake shall be attached at intervals of 5 feet 0 inch (1524 mm).

Attachment for continuous ties shall be nails, screws staples or approved clips of the same material as the ties, and shall not be subjected to withdrawal forces. Attachments for continuous ties shall have an allowable working stress shear resistance of not less than twice the dead weight of the tile tributary to the attachment, but not less than 300 pounds (136 kg).

- Tile with projecting anchor lugs at the bottom of the tiles shall be held in position by means of 1- inch by 2- inch (25mm by 51mm) wood stripping nailed to the roof sheathing over the underlay.

- Clay or concrete tile on roofs with slopes exceeding 24 units vertical in 12 units horizontal (200 percent slope) shall be attached as required for veneer in Chapter 14. The nose of all tiles shall be securely fastened.

- Clay or concrete tile shall have a minimum of two fasteners per tile. Tiles that are 8 inches (203 mm) in width or less are permitted to be fastened at the center of the head with one fastener per tile.

- Interlocking clay or concrete tile shall have a minimum of one nail near center of head or two wire ties per tile.

1513.5 Slate Shingles

Slate shingles on roofs with slopes exceeding 24 units vertical in 12 units horizontal (200 percent slope) shall be attached as required for veneer per Chapter 14.

[BS] Adhered Masonry Veneer

Veneer secured and supported through the adhesion of an approved bonding material applied to an approved backing.